| Aromas Cubanos Manufaktúra | Vizsgaremek dokumentáció 2024/2025. tanév 13.E osztály Munkatársak Balázs Zalán Dobó Zoltán Dávid Jelenovits Milán |
| --- | --- |

Tartalomjegyzék

Feladat megtervezése és topológia összeállítása	3

Topológia összeállítása	6

Használt hálózati eszközök	8

IPv4 címzési rendszer (VLSM)	9

IPv6 címzési rendszer (VLSM)	15

IP címek konfigurációja	18

IPv6 címek konfigurációja	29

Második rétegbeli redundancia megvalósítása	30

Harmadik rétegbeli redundancia megvalósítása	34

Vezetéknélküli hálózat konfigurációja	36

Statikus forgalomirányítás konfigurációja.	40

Dinamikus forgalomirányítás konfigurációja	41

Statikus címfordítás megvalósítása	42

Dinamikus címfordítás megvalósítása	43

WAN összeköttetés	47

VPN kapcsolat megvalósítása	48

Hálózatkonfigurációs program	50

ACL-ek	56

ASA tűzfal konfigurációja	57

NTP és Syslog konfigurációja	58

Hálózati eszközök teljes konfigurációja	59

Windows szerver telepítése virtuális gépre	90

Linux szerver telepítése virtuális gépre	91

Cisco Packet Tracer Tesztelés	102

Oracle VM VirtualBox tesztelés	113

Felhasznált szoftverek	116

Ábrajegyzék	117

Feladat megtervezése és topológia összeállítása

Az Aromas Cubanos projekt felépítése és célja

Történet: Az Aromas Cubanos egy ambiciózus vállalkozás, amely 1975-ben alakult a kubai szivargyártás szívében. Az alapítók egy családi örökséget kívántak tovább vinni, az autentikus kubai szivarok kézműves gyártását, amelyeket világszerte elismertek. A cég dinamikusan fejlődött, és az évtizedek során jelentős mértékben bővítette kapacitásait. 2023-ra az Aromas Cubanos három fő telephelyet és egy kávézót is magában foglaló hálózatot épített ki, amely a modern technológia és a hagyományos szivargyártás egyedülálló ötvözetét nyújtja.

Projekt célja: Az Aromas Cubanos legújabb projektje egy olyan komplex hálózati infrastruktúra létrehozása, amely támogatja a cég bővülését és hatékony működését. A cél egy biztonságos és gyors, mindhárom telephelyet lefedő hálózat kialakítása, amely kielégíti a cég működési igényeit és a modern IT-követelményeket. Az infrastruktúrának több VLAN-t, vezeték nélküli hálózatokat, redundáns megoldásokat, valamint statikus és dinamikus forgalomirányítást is tartalmaznia kell.

Hálózat felépítése: Az infrastruktúra négy fő telephelyet fed le, ahol a szivargyártás, kutatás-fejlesztés, az irodai adminisztráció és egy kávézó szolgáltatás zajlik. Minden helyszínen különböző VLAN-ok kerültek kialakításra a dolgozók, vezetőség és vendégek elkülönítésére. A gyártási részlegen különösen fontos a biztonságos kommunikáció biztosítása a fejlesztési részleggel és a vezetőséggel, ezért a redundáns megoldások mellett tűzfalszabályok (ACL-ek) és VPN-kapcsolatok is beépítésre kerültek.

A hálózat mind IPv4, mind IPv6 címzési rendszert alkalmaz, lehetővé téve a zökkenőmentes adatkommunikációt és skálázhatóságot a jövőbeli bővítésekhez. A statikus és dinamikus címfordítás, valamint a WAN-összeköttetések biztosítják a telephelyek közötti zavartalan adatforgalmat.

Biztonság és programozhatóság: A hálózat tartalmaz vezeték nélküli hozzáférési pontokat (WiFi), mind nyílt, mind zárt hálózatok számára, biztosítva a vendégek kényelmét a kávézóban, miközben a vállalat belső adatforgalma védve van. A biztonság fokozása érdekében hardveres tűzfalak és ACL-ek szabályozzák az adatforgalmat.

A rendszergazdák központilag felügyelik a hálózatot, programozott hálózatkonfigurációk segítségével, automatikusan menedzselve a mentéseket és szoftvertelepítéseket. A rendszer része két kiszolgáló, egy Linux alapú és egy Windows szerver, amelyek olyan alapvető szolgáltatásokat nyújtanak, mint az Active Directory, DHCP, DNS, HTTPS, fájl- és nyomtatómegosztás, valamint automatizált biztonsági mentések.

Az Aromas Cubanos célja, hogy a technológiai újításokat és hagyományos szivarkészítési módszereit egyaránt fenntartsa, hozzájárulva ezzel a kubai szivarok globális hírnevének megőrzéséhez és terjesztéséhez.

Vizsgaremek alapterv

Téma: 	Kézműves szivar cég kávézóval

Név: 	Aromas Cubanos (Kubai Ízek)

Helyszín: 	Kuba

Épületek: 	Gyártó ház és Raktár, Fejlesztési részleg, Vezetői/Hálózati Központ, Kávézó

Gyártó ház és Raktár (Raktár)

Szervezeti egységek:

Dolgozók	50 fő

Security	2 fő

Közép-Vezetés	1 fő

Eszközök:

WiFi Access Point,

Számítógép,

Mobiltelefon,

Forgalomirányító,

Kapcsoló,

Hálózati Kontroller

Fejlesztési részleg (Fejlesztés)

Szervezeti egységek:

Dolgozók	20 fő

Security	2 fő

Közép-Vezetés	1 fő

Eszközök:

WiFi Access Point,

Számítógép,

Mobiltelefon,

Forgalomirányító,

Kapcsoló

ASA Tűzfal

Vezetői/Hálózati Központ (Vezetőség)

Szervezeti egységek:

Irodisták	10 fő

Security	2 fő

CEO	5 fő

Rendszergazda	2 fő

Eszközök:

Számítógép,

Kiszolgáló,

Forgalomirányító,

Kapcsoló

Kávézó

Szervezeti egységek:

Dolgozók	6 fő

Vendégek	30 fő

Közép-Vezetés	1 fő

Eszközök:

Nyilvános WiFi Access Point,

Dolgozói WiFi Access Point,

Számítógép,

Mobiltelefon,

Forgalomirányító,

Kapcsoló

Topológia összeállítása

1. ábra: Teljes topológia képe

2. ábra: Raktár topológia

3. ábra: Kávézó topológia

4. ábra: Fejlesztés topológia

5. ábra: Vezetőség topológia

Használt hálózati eszközök

Vezetőség épület

Router 1: Cisco ISR4331-es forgalomirányító, „RV”, internetelérést biztosít, VPN elérést biztosít, HSRP protokollt használ redundanciához

Router 2: Cisco 2911-es forgalomirányító, „RV-2”, WAN kapcsolatot illetve másodlagos internetelérést biztosít, HSRP protokollt használ redundanciához

Switch 1: Cisco 2960-24TT kapcsoló, „SW V”, a számítógépek hálózati kapcsolatáért felel, második rétegbeli redundanciával állandó elérésért felel

Switch 2: Cisco 2960-24TT kapcsoló, „SW V-2”, a számítógépek hálózati kapcsolatáért felel, második rétegbeli redundanciával állandó elérésért felel

Switch 3: Cisco 2960-24TT kapcsoló, „SW V-3”, a számítógépek hálózati kapcsolatáért felel, második rétegbeli redundanciával állandó elérésért felel

Raktár épület

Router 1: Cisco 2911-es forgalomirányító, „RR”, internetelérést biztosít

Switch 1: Cisco 2960-24TT kapcsoló, „SW R”, a számítógépek hálózati kapcsolatáért felel

Access Point 1: Cisco AP, „AP-R”, vezetéknélküli internetelérést biztosít a dolgozók számára
Network Controller 1: Cisco PT Controller, „NetworkController”, Packet Tracer-ben szimulált hálózatprogramozás lehetőségéért felel

Fejlesztés épület

ASA 1: Cisco ASA tűzfal, „ASA”, fizikai tűzfal, OSPF-fel fedezi fel a hálózatot

Router 1: Cisco 2911-es forgalomirányító, „RF”, internetelérést biztosít, OSPF-fel fedezi fel a hálózatot

Router 2: Cisco 2911-es forgalomirányító, „RF-2”, WAN kapcsolatot illetve másodlagos internetelérést biztosít, OSPF-fel fedezi fel a hálózatot

Switch 1: Cisco 2960-24TT kapcsoló, „SW F”, a számítógépek hálózati kapcsolatáért felel

Switch 2: Cisco 2960-24TT kapcsoló, „SW F-2”, a számítógépek hálózati kapcsolatáért felel

Access Point 1: Cisco AP, „AP-F”, vezetéknélküli internetelérést biztosít a dolgozók számára

Kávézó épület

Router 1: Cisco 2911-es forgalomirányító, „RK”, internetelérést biztosít

Switch 1: Cisco 2960-24TT kapcsoló, „SW K”, a számítógépek hálózati kapcsolatáért felel

Access Point 1: Cisco AP, „AP-K”, vezetéknélküli internetelérést biztosít a dolgozók számára

Access Point 2: Cisco AP, „AP-KG”, vezetéknélküli internetelérést biztosít a vendégek számára

IPv4 címzési rendszer (VLSM)

Címmagyarázat

| Címmagyarázat |  |  |  |
| --- | --- | --- | --- |
| Hálózat | VLAN ID | Épület | Eszköz |
| 10 | 1 | 2 | 1 |

| Épület kódok |  |
| --- | --- |
| 1 | Vezetőség |
| 2 | Raktár |
| 3 | Fejlesztés |
| 4 | Kávézó |

Dolgozók VLAN

| 10.10.2. | 0 | Dolgozók VLAN | Hálózati cím |
| --- | --- | --- | --- |
| 10.10.2. | 1 |  |  |
| 10.10.2. | 2 |  |  |
| 10.10.2. | 3 |  |  |
| 10.10.2. | 4 |  |  |
| 10.10.2. | 5 |  |  |
| 10.10.2. | 6 |  |  |
| 10.10.2. | 7 |  |  |
| 10.10.2. | 8 |  |  |
| 10.10.2. | 9 |  |  |
| 10.10.2. | 10 |  |  |
| 10.10.2. | 11 |  |  |
| 10.10.2. | 12 |  |  |
| 10.10.2. | 13 |  |  |
| 10.10.2. | 14 |  |  |
| 10.10.2. | 15 |  |  |
| 10.10.2. | 16 |  |  |
| 10.10.2. | 17 |  |  |
| 10.10.2. | 18 |  |  |
| 10.10.2. | 19 |  |  |
| 10.10.2. | 20 |  |  |
| 10.10.2. | 21 |  |  |
| 10.10.2. | 22 |  |  |
| 10.10.2. | 23 |  |  |
| 10.10.2. | 24 |  |  |
| 10.10.2. | 25 |  |  |
| 10.10.2. | 26 |  |  |

Dolgozók tábla (részlet)

Vendégek VLAN

| 10.20.4. | 0 | Vendégek VLAN | Hálózati cím |
| --- | --- | --- | --- |
| 10.20.4. | 1 |  |  |
| 10.20.4. | 2 |  |  |
| 10.20.4. | 3 |  |  |
| 10.20.4. | 4 |  |  |
| 10.20.4. | 5 |  |  |
| 10.20.4. | 6 |  |  |
| 10.20.4. | 7 |  |  |
| 10.20.4. | 8 |  |  |
| 10.20.4. | 9 |  |  |
| 10.20.4. | 10 |  |  |
| 10.20.4. | 11 |  |  |
| 10.20.4. | 12 |  |  |
| 10.20.4. | 13 |  |  |
| 10.20.4. | 14 |  |  |
| 10.20.4. | 15 |  |  |
| 10.20.4. | 16 |  |  |
| 10.20.4. | 17 |  |  |
| 10.20.4. | 18 |  |  |
| 10.20.4. | 19 |  |  |
| 10.20.4. | 20 |  |  |
| 10.20.4. | 21 |  |  |
| 10.20.4. | 22 |  |  |
| 10.20.4. | 23 |  |  |
| 10.20.4. | 24 |  |  |
| 10.20.4. | 25 |  |  |
| 10.20.4. | 26 |  |  |
| 10.20.4. | 27 |  |  |
| 10.20.4. | 28 |  |  |
| 10.20.4. | 29 |  |  |
| 10.20.4. | 30 |  |  |
| 10.20.4. | 254 |  | Gateway |
| 10.20.4. | 255 |  | Szórási cím |

Vendégek tábla

Irodisták VLAN

| 10.30.1. | 0 | Irodisták VLAN | Hálózati cím |
| --- | --- | --- | --- |
| 10.30.1. | 1 |  |  |
| 10.30.1. | 2 |  |  |
| 10.30.1. | 3 |  |  |
| 10.30.1. | 4 |  |  |
| 10.30.1. | 5 |  |  |
| 10.30.1. | 6 |  |  |
| 10.30.1. | 7 |  |  |
| 10.30.1. | 8 |  |  |
| 10.30.1. | 9 |  |  |
| 10.30.1. | 10 |  |  |
| 10.30.1 | 252 |  | Gateway RV2 |
| 10.30.1 | 253 |  | Gateway RV |
| 10.30.1. | 254 |  | HSRP GW |
| 10.30.1. | 255 |  | Szórási cím |

Irodisták tábla

Security VLAN

| 10.40.1. | 0 | Security | Hálózati cím |
| --- | --- | --- | --- |
| 10.40.1. | 1 |  |  |
| 10.40.1. | 2 |  |  |
| 10.40.1. | 3 |  |  |
| 10.40.1. | 4 |  |  |
| 10.40.1 | 252 |  | Gateway RV2 |
| 10.40.1 | 253 |  | Gateway RV |
| 10.40.1. | 254 |  | HSRP GW |
| 10.40.1. | 255 |  | Szórási cím |
| 10.40.2. | 0 |  | Hálózati cím |
| 10.40.2. | 1 |  |  |
| 10.40.2. | 2 |  |  |
| 10.40.2. | 3 |  |  |
| 10.40.2. | 4 |  |  |
| 10.40.2. | 254 |  | Gateway |
| 10.40.2. | 255 |  | Szórási cím |
| 10.40.3. | 0 |  | Hálózati cím |
| 10.40.3. | 1 |  |  |
| 10.40.3. | 2 |  |  |
| 10.40.3. | 3 |  |  |
| 10.40.3. | 4 |  |  |
| 10.40.3. | 254 |  | Gateway |
| 10.40.3. | 255 |  | Szórási cím |

Security tábla

CEO VLAN

| 10.50.1. | 0 | CEO | Hálózati cím |
| --- | --- | --- | --- |
| 10.50.1. | 1 |  |  |
| 10.50.1. | 2 |  |  |
| 10.50.1. | 3 |  |  |
| 10.50.1. | 4 |  |  |
| 10.50.1. | 5 |  |  |
| 10.50.1 | 252 |  | Gateway RV2 |
| 10.50.1 | 253 |  | Gateway RV |
| 10.50.1. | 254 |  | HSRP GW |
| 10.50.1. | 255 |  | Szórási cím |

CEO tábla

Közép-Vezetés VLAN

| 10.60.2. | 0 | Közép-Vezetés | Hálózati cím |
| --- | --- | --- | --- |
| 10.60.2. | 1 |  |  |
| 10.60.2. | 2 |  |  |
| 10.60.2. | 254 |  | Gateway |
| 10.60.2. | 255 |  | Szórási cím |
| 10.60.3. | 0 |  | Hálózati cím |
| 10.60.3. | 1 |  |  |
| 10.60.3. | 2 |  |  |
| 10.60.3. | 254 |  | Gateway |
| 10.60.3. | 255 |  | Szórási cím |
| 10.60.4. | 0 |  | Hálózati cím |
| 10.60.4. | 1 |  |  |
| 10.60.4. | 2 |  |  |
| 10.60.4. | 254 |  | Gateway |
| 10.60.4. | 255 |  | Szórási cím |

Közép-Vezetés tábla

Rendszergazda VLAN

| 10.70.1. | 0 | Admin | Hálózati cím |
| --- | --- | --- | --- |
| 10.70.1. | 1 |  | Irodai Gép 1 |
| 10.70.1. | 2 |  | Irodai Gép 2 |
| 10.70.1 | 252 |  | Gateway RV2 |
| 10.70.1 | 253 |  | Gateway RV |
| 10.70.1. | 254 |  | HSRP GW |
| 10.70.1. | 255 |  | Szórási cím |

Rendszergazda tábla

Szerverek VLAN

| 10.80.1. | 0 | SRV | Hálózati cím |
| --- | --- | --- | --- |
| 10.80.1. | 1 |  | Windows Server |
| 10.80.1. | 2 |  | Linux Server |
| 10.80.1 | 252 |  | Gateway RV2 |
| 10.80.1 | 253 |  | Gateway RV |
| 10.80.1. | 254 |  | HSRP GW |
| 10.80.1. | 255 |  | Szórási cím |

Szerverek tábla

WAN VLAN

| 10.0.0. | 0 | WAN | Hálózati cím |
| --- | --- | --- | --- |
| 10.0.0. | 1 |  | RV-2 WAN |
| 10.0.0. | 2 |  | RF-2 WAN |
| 10.0.0. | 3 |  | Szórási cím |

WAN tábla

Loopback VLAN

| 10.200.1. | 1 | Loopback | RV Lo0 | /32 |
| --- | --- | --- | --- | --- |
| 10.200.1. | 2 |  | RV-2 Lo0 | /32 |
| 10.200.2. | 1 |  | RR Lo0 | /32 |
| 10.200.3. | 1 |  | RF Lo0 | /32 |
| 10.200.3. | 2 |  | RF-2 Lo0 | /32 |
| 10.200.4. | 1 |  | RK Lo0 | /32 |

Loopback tábla

ASA VLAN

| 10.201.0. | 0 | WAN | Hálózati cím |
| --- | --- | --- | --- |
| 10.201.0. | 1 |  | RF Gig0/2 |
| 10.201.0. | 2 |  | ASA |
| 10.201.0. | 3 |  | Szórási cím |

WAN tábla

Management VLAN

| 10.100.1. | 0 | Management VLAN | Alhálózati cím |
| --- | --- | --- | --- |
| 10.100.1. | 1 |  | Sw V Vlan 1 |
| 10.100.1. | 2 |  | Sw V-2 Vlan 1 |
| 10.100.1. | 3 |  | Sw V-3 Vlan 1 |
| 10.100.1. | 4 |  | RV Gig0/1 |
| 10.100.1. | 5 |  | RV-2 Gig0/1 |
| 10.100.1. | 252 |  | Gateway RV2 |
| 10.100.1. | 253 |  | Gateway RV |
| 10.100.1. | 254 |  | HSRP GW |
| 10.100.1. | 255 |  | Szórási cím |
| 10.100.2. | 0 |  | Hálózati cím |
| 10.100.2. | 1 |  | RR Gig0/1 |
| 10.100.2. | 2 |  | Sw R VLAN 1 |
| 10.100.2. | 254 |  | Gateway |
| 10.100.2. | 255 |  | Szórási cím |
| 10.100.3. | 0 |  | Alhálózati cím |
| 10.100.3. | 1 |  | RF Gig0/0 |
| 10.100.3. | 2 |  | RF-2 Gig0/2 |
| 10.100.3. | 3 |  |  |
| 10.100.3. | 4 |  |  |
| 10.100.3. | 5 |  |  |
| 10.100.3. | 6 |  |  |
| 10.100.3. | 7 |  | Szórási cím |
| 10.100.3. | 8 |  | Alhálózati cím |
| 10.100.3. | 9 |  | RF Gig0/1 |
| 10.100.3. | 10 |  | RF-2 Gig0/0 |
| 10.100.3. | 11 |  |  |
| 10.100.3. | 12 |  |  |
| 10.100.3. | 13 |  |  |
| 10.100.3. | 14 |  |  |
| 10.100.3. | 15 |  | Szórási cím |
| 10.100.3. | 16 |  | Alhálózati cím |
| 10.100.3. | 17 |  | Sw F VLAN 1 |
| 10.100.3. | 18 |  | Sw F-2 VLAN 1 |
| 10.100.3. | 19 |  |  |
| 10.100.3. | 20 |  |  |
| 10.100.3. | 21 |  |  |
| 10.100.3. | 22 |  |  |
| 10.100.3. | 23 |  | Szórási cím |

| 10.100.4. | 0 |  | Hálózati cím |
| --- | --- | --- | --- |
| 10.100.4. | 1 |  | RK Gig0/1 |
| 10.100.4. | 2 |  | Sw K VLAN 1 |
| 10.100.4. | 3 |  |  |
| 10.100.4. | 254 |  | Gateway |
| 10.100.4. | 255 |  | Szórási cím |

Management tábla

IPv6 címzési rendszer (VLSM)

Címmagyarázat

| Cím magyarázat |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Hálózat |  |  | VLAN ID | Épület ID | Eszköz |
| 2001 | db8 | 1984 | 1 | 02 | 1 |

| Épület kódok |  |
| --- | --- |
| 01 | Vezetőség |
| 02 | Raktár |
| 03 | Fejlesztés |
| 04 | Kávézó |

Dolgozók VLAN

| 2001:db8:1984:102:: |  | Dolgozók VLAN | Hálózati cím |
| --- | --- | --- | --- |
| 2001:db8:1984:102:: | 1 |  |  |
| 2001:db8:1984:102:: | 2 |  |  |
| 2001:db8:1984:102:: | 3 |  |  |
| 2001:db8:1984:102:: | 4 |  |  |
| 2001:db8:1984:102:: | 5 |  |  |
| 2001:db8:1984:102:: | 6 |  |  |
| 2001:db8:1984:102:: | 7 |  |  |
| 2001:db8:1984:102:: | 8 |  |  |
| 2001:db8:1984:102:: | 9 |  |  |
| 2001:db8:1984:102:: | A |  |  |
| 2001:db8:1984:102:: | B |  |  |
| 2001:db8:1984:102:: | C |  |  |
| 2001:db8:1984:102:: | D |  |  |
| 2001:db8:1984:102:: | E |  |  |
| 2001:db8:1984:102:: | F |  |  |
| 2001:db8:1984:102:: | 10 |  |  |
| 2001:db8:1984:102:: | 11 |  |  |
| 2001:db8:1984:102:: | 12 |  |  |
| 2001:db8:1984:102:: | 13 |  |  |
| 2001:db8:1984:102:: | 14 |  |  |
| 2001:db8:1984:102:: | 15 |  |  |
| 2001:db8:1984:102:: | 16 |  |  |
| 2001:db8:1984:102:: | 17 |  |  |
| 2001:db8:1984:102:: | 18 |  |  |
| 2001:db8:1984:102:: | 19 |  |  |
| 2001:db8:1984:102:: | 1A |  |  |
| 2001:db8:1984:102:: | 1B |  |  |
| 2001:db8:1984:102:: | 1C |  |  |
| 2001:db8:1984:102:: | 1D |  |  |
| 2001:db8:1984:102:: | 1E |  |  |
| 2001:db8:1984:102:: | 1F |  |  |

Dolgozók tábla (részlet)

Vendégek VLAN

| 2001:db8:1984:204:: |  | Vendégek VLAN | Hálózati cím |
| --- | --- | --- | --- |
| 2001:db8:1984:204:: | 1 |  |  |
| 2001:db8:1984:204:: | 2 |  |  |
| 2001:db8:1984:204:: | 3 |  |  |
| 2001:db8:1984:204:: | 4 |  |  |
| 2001:db8:1984:204:: | 5 |  |  |
| 2001:db8:1984:204:: | 6 |  |  |
| 2001:db8:1984:204:: | 7 |  |  |
| 2001:db8:1984:204:: | 8 |  |  |
| 2001:db8:1984:204:: | 9 |  |  |
| 2001:db8:1984:204:: | A |  |  |
| 2001:db8:1984:204:: | B |  |  |
| 2001:db8:1984:204:: | C |  |  |
| 2001:db8:1984:204:: | D |  |  |
| 2001:db8:1984:204:: | E |  |  |
| 2001:db8:1984:204:: | F |  |  |
| 2001:db8:1984:204:: | 10 |  |  |
| 2001:db8:1984:204:: | 11 |  |  |
| 2001:db8:1984:204:: | 12 |  |  |
| 2001:db8:1984:204:: | 13 |  |  |
| 2001:db8:1984:204:: | 14 |  |  |
| 2001:db8:1984:204:: | 15 |  |  |
| 2001:db8:1984:204:: | 16 |  |  |
| 2001:db8:1984:204:: | 17 |  |  |
| 2001:db8:1984:204:: | 18 |  |  |
| 2001:db8:1984:204:: | 19 |  |  |
| 2001:db8:1984:204:: | 1A |  |  |
| 2001:db8:1984:204:: | 1B |  |  |
| 2001:db8:1984:204:: | 1C |  |  |
| 2001:db8:1984:204:: | 1D |  |  |
| 2001:db8:1984:204:: | 1E |  |  |
| 2001:db8:1984:204:: | 1F |  |  |

Vendégek tábla

Irodisták VLAN

| 2001:db8:1984:301:: |  | Irodisták VLAN | Hálózati cím |
| --- | --- | --- | --- |
| 2001:db8:1984:301:: | 1 |  |  |
| 2001:db8:1984:301:: | 2 |  |  |
| 2001:db8:1984:301:: | 3 |  |  |
| 2001:db8:1984:301:: | 4 |  |  |
| 2001:db8:1984:301:: | 5 |  |  |
| 2001:db8:1984:301:: | 6 |  |  |
| 2001:db8:1984:301:: | 7 |  |  |
| 2001:db8:1984:301:: | 8 |  |  |
| 2001:db8:1984:301:: | 9 |  |  |
| 2001:db8:1984:301:: | A |  |  |

Irodisták tábla

Security VLAN

| 2001:db8:1984:401:: |  | Security | Hálózati cím |
| --- | --- | --- | --- |
| 2001:db8:1984:401:: | 1 |  |  |
| 2001:db8:1984:401:: | 2 |  |  |
| 2001:db8:1984:401:: | 3 |  |  |
| 2001:db8:1984:401:: | 4 |  |  |
| 2001:db8:1984:402:: |  |  | Hálózati cím |
| 2001:db8:1984:402:: | 1 |  |  |
| 2001:db8:1984:402:: | 2 |  |  |
| 2001:db8:1984:402:: | 3 |  |  |
| 2001:db8:1984:402:: | 4 |  |  |
| 2001:db8:1984:403:: |  |  | Hálózati cím |
| 2001:db8:1984:403:: | 1 |  |  |
| 2001:db8:1984:403:: | 2 |  |  |
| 2001:db8:1984:403:: | 3 |  |  |
| 2001:db8:1984:403:: | 4 |  |  |

Security tábla

CEO VLAN

| 2001:db8:1984:501:: |  | CEO | Hálózati cím |
| --- | --- | --- | --- |
| 2001:db8:1984:501:: | 1 |  |  |
| 2001:db8:1984:501:: | 2 |  |  |
| 2001:db8:1984:501:: | 3 |  |  |
| 2001:db8:1984:501:: | 4 |  |  |
| 2001:db8:1984:501:: | 5 |  |  |

CEO tábla

Közép-Vezetés VLAN

| 2001:db8:1984:602:: |  | Közép-Vezetés | Hálózati cím |
| --- | --- | --- | --- |
| 2001:db8:1984:602:: | 1 |  |  |
| 2001:db8:1984:602:: | 2 |  |  |
| 2001:db8:1984:603:: |  |  | Hálózati cím |
| 2001:db8:1984:603:: | 1 |  |  |
| 2001:db8:1984:603:: | 2 |  |  |
| 2001:db8:1984:604:: |  |  | Hálózati cím |
| 2001:db8:1984:604:: | 1 |  |  |
| 2001:db8:1984:604:: | 2 |  |  |

Közép-Vezetés tábla

Rendszergazda VLAN

| 2001:db8:1984:701:: |  | Admin | Hálózati cím |
| --- | --- | --- | --- |
| 2001:db8:1984:701:: | 1 |  | Irodai Gép 1 |
| 2001:db8:1984:701:: | 2 |  | Irodai Gép 2 |
| 2001:db8:1984:701:: | 3 |  |  |
| 2001:db8:1984:701:: | 4 |  |  |

Rendszergazda tábla

Szerverek VLAN

| 2001:db8:1984:801:: |  | SRV | Hálózati cím |
| --- | --- | --- | --- |
| 2001:db8:1984:801:: | 1 |  | Windows Server |
| 2001:db8:1984:801:: | 2 |  | Linux Server |

Szerverek tábla

Management VLAN

| 2001:db8:1984:1001:: | 0 | Management VLAN | Hálózati cím |
| --- | --- | --- | --- |
| 2001:db8:1984:1001:: | 1 |  | Sw V Vlan 1 |
| 2001:db8:1984:1001:: | 2 |  | Sw V-2 Vlan 1 |
| 2001:db8:1984:1001:: | 3 |  | Sw V-3 Vlan 1 |
| 2001:db8:1984:1002:: | 0 |  | Hálózati cím |
| 2001:db8:1984:1002:: | 1 |  | RR Gig0/1 |
| 2001:db8:1984:1002:: | 2 |  | Sw R VLAN 1 |
| 2001:db8:1984:1002:: | 3 |  |  |
| 2001:db8:1984:1002:: | 4 |  |  |
| 2001:db8:1984:1003:: | 0 |  | Hálózati cím |
| 2001:db8:1984:1003:: | 1 |  | RF Gig0/0 |
| 2001:db8:1984:1003:: | 2 |  | RF-2 Gig0/2 |
| 2001:db8:1984:1003:: | 3 |  |  |
| 2001:db8:1984:1003:: | 4 |  |  |
| 2001:db8:1984:1003:: | 5 |  |  |
| 2001:db8:1984:1003:: | 6 |  |  |
| 2001:db8:1984:1003:: | 7 |  |  |
| 2001:db8:1984:1003:: | 8 |  |  |
| 2001:db8:1984:1003:: | 9 |  | RF Gig0/1 |
| 2001:db8:1984:1003:: | A |  | RF-2 Gig0/0 |
| 2001:db8:1984:1003:: | B |  |  |
| 2001:db8:1984:1003:: | C |  |  |
| 2001:db8:1984:1003:: | D |  |  |
| 2001:db8:1984:1003:: | E |  |  |
| 2001:db8:1984:1003:: | F |  |  |
| 2001:db8:1984:1003:: | 11 |  |  |
| 2001:db8:1984:1003:: | 12 |  | Sw F VLAN 1 |
| 2001:db8:1984:1003:: | 13 |  | Sw F-2 VLAN 1 |
| 2001:db8:1984:1003:: | 14 |  |  |
| 2001:db8:1984:1003:: | 15 |  |  |
| 2001:db8:1984:1003:: | 16 |  |  |
| 2001:db8:1984:1003:: | 17 |  |  |
| 2001:db8:1984:1004:: | 0 |  | Hálózati cím |
| 2001:db8:1984:1004:: | 1 |  | RK Gig0/1 |
| 2001:db8:1984:1004:: | 2 |  | Sw K VLAN 1 |
| 2001:db8:1984:1004:: | 3 |  |  |

Management tábla

WAN VLAN

| 2001:db8:1984:: |  | WAN | Hálózati cím |
| --- | --- | --- | --- |
| 2001:db8:1984:: | 1 |  | RV-2 WAN |
| 2001:db8:1984:: | 2 |  | RF-2 WAN |

WAN tábla

IP címek konfigurációja

VLAN tábla

| Vlan száma | Vlan neve |
| --- | --- |
| 10 | Dolgozók |
| 20 | Vendégek |
| 30 | Irodisták |
| 40 | Security |
| 50 | CEO |
| 60 | Közép-Vezetés |
| 70 | Admin |
| 80 | SRV |
| 100 | Management |
| 200 | Loopback |

Vezetőség épület

RV forgalomirányító

interface gig0/0/1

description RV-SwV

ip address 10.100.1.253 255.255.255.0

ip helper-address 10.80.1.2

no shutdown

exit

!

interface gig0/0/2

description ISP

ip address 200.100.0.18 255.255.255.248

no shutdown

exit

!

interface lo0

description Router-ID

ip address 10.200.1.1 255.255.255.255

no shutdown

exit

!

interface gig0/0/1.30

description V-Irodistak-GW

encapsulation dot1q 30

ip address 10.30.1.253 255.255.255.0

ip helper-address 10.80.1.2

no shutdown

exit

!

interface gig0/0/1.40

description V-Security-GW

encapsulation dot1q 40

ip address 10.40.1.253 255.255.255.0

ip helper-address 10.80.1.2

no shutdown

exit

!

interface gig0/0/1.50

description V-CEO-GW

encapsulation dot1q 50

ip address 10.50.1.253 255.255.255.0

ip helper-address 10.80.1.2

ip nat inside

no shutdown

exit

!

interface gig0/0/1.70

description V-Admin-GW

encapsulation dot1q 70

ip address 10.70.1.253 255.255.255.0

ip helper-address 10.80.1.2

ip nat inside

no shutdown

exit

!

interface gig0/0/1.80

description V-SRV-GW

encapsulation dot1q 80

ip address 10.80.1.253 255.255.255.0

ip helper-address 10.80.1.2

ip nat inside

no shutdown

exit

RV-2 forgalomirányító

interface gig0/1

description RV2-SwV3

ip address 10.100.1.5 255.255.255.0

no shutdown

exit

!

interface gig0/0/0

description WAN

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

!

interface lo0

description Router-ID

ip address 10.200.1.2 255.255.255.248

no shutdown

exit

!

interface gig0/1.30

description V-Irodistak-GW

encapsulation dot1q 30

ip address 10.30.1.252 255.255.255.0

no shutdown

exit

!

interface gig0/1.40

description V-Security-GW

encapsulation dot1q 40

ip address 10.40.1.252 255.255.255.0

no shutdown

exit

!

interface gig0/1.50

description V-CEO-GW

encapsulation dot1q 50

ip address 10.50.1.252 255.255.255.0

no shutdown

exit

!

interface gig0/1.70

description V-Admin-GW

encapsulation dot1q 70

ip address 10.70.1.252 255.255.255.0

no shutdown

exit

!

interface gig0/1.80

description V-SRV-GW

encapsulation dot1q 80

ip address 10.80.1.252 255.255.255.0

no shutdown

exit

SW V kapcsoló

vlan 30

name V-Irodistak

vlan 40

name V-Security

vlan 50

name V-CEO

vlan 70

name V-Rendszergazda

vlan 80

name V-SERVER

vlan 100

name V-Mngmnt

exit

!

interface vlan 1

description Mngmnt

ip address 10.100.1.1 255.255.255.0

no shutdown

SW V-2 kapcsoló

vlan 30

name V-Irodistak

vlan 40

name V-Security

vlan 50

name V-CEO

vlan 70

name V-Rendszergazda

vlan 80

name V-SERVER

vlan 100

name V-Mngmnt

exit

!

interface vlan 1

description Mngmnt

ip address 10.100.1.2 255.255.255.0

no shutdown

SW V-3 kapcsoló

vlan 30

name V-Irodistak

vlan 40

name V-Security

vlan 50

name V-CEO

vlan 70

name V-Rendszergazda

vlan 80

name V-SERVER

vlan 100

name V-Mngmnt

exit

!

interface vlan 1

description Mngmnt

ip address 10.100.1.3 255.255.255.0

no shutdown

Raktár épület

RR forgalomirányító

interface gig0/1

description RR-SwR

ip address 10.100.2.1 255.255.255.252

no shutdown

exit

!

interface gig0/1.60

description R-Vezetoseg-GW

encapsulation dot1q 60

ip address 10.60.2.254 255.255.255.0

exit

!

interface gig0/1.40

description R-Security-GW

encapsulation dot1q 40

ip address 10.40.2.254 255.255.255.0

exit

!

interface gig0/0/0

description ISP

ip address 200.100.0.1 255.255.255.252

no shutdown

exit

!

interface gig0/0

description WiFi-GW

ip address 10.10.2.254 255.255.255.0

no shutdown

exit

!

ip dhcp excluded-address 10.10.2.254

ip dhcp excluded-address 10.40.2.254

ip dhcp excluded-address 10.60.2.254

ip dhcp pool R-Vezetoseg

dns 200.100.0.20

lease 0 1 30

default 10.60.2.254

network 10.60.2.0 255.255.255.0

exit

!

ip dhcp pool R-Security

dns 200.100.0.20

lease 0 1 30

default 10.40.2.254

network 10.40.2.0 255.255.255.0

exit

!

ip dhcp pool R-WiFi

dns 200.100.0.20

lease 0 1 30

default 10.10.2.254

network 10.10.2.0 255.255.255.0

exit

!

interface lo0

description Router-ID

ip address 10.200.2.1 255.255.255.255

no shutdown

SW R kapcsoló

vlan 40

name R-Security

vlan 60

name R-Vezetoseg

exit

!

interface vlan 1

description Mngmnt

ip address 10.100.2.2 255.255.255.252

Fejlesztés épület

ASA tűzfal

interface gig1/1

nameif inside

security-level 100

ip address 10.0.0.2 255.0.0.0

no shutdown

!

interface gig1/2

nameif outside

security-level 0

ip address 200.100.0.9 255.255.255.252

no shutdown

exit

RF forgalomirányító

interface gig0/1

description RF-SwF

ip address 10.100.3.5 255.255.255.252

no shutdown

exit

!

interface gig0/0

description RF-RF2

ip address 10.100.3.1 255.255.255.252

no shutdown

exit

!

interface gig0/0/0

description ISP

ip address 200.100.0.9 255.255.255.252

no shutdown

exit

!

interface lo0

description Router-ID

ip address 10.200.3.1 255.255.255.255

no shutdown

exit

!

interface gig0/1.10

description F-Dolgozok-GW

encapsulation dot1q 10

ip address 10.10.3.126 255.255.255.128

no shutdown

exit

!

interface gig0/1.40

description F-Security-GW

encapsulation dot1q 40

ip address 10.40.3.254 255.255.255.0

no shutdown

exit

!

interface gig0/1.60

description F-Vezetoseg-GW

encapsulation dot1q 60

ip address 10.60.3.254 255.255.255.0

no shutdown

exit

!

ip dhcp excluded-address 10.10.3.126

ip dhcp excluded-address 10.40.3.254

ip dhcp excluded-address 10.60.3.254

!

ip dhcp pool F-Dolgozok

dns 200.100.0.20

lease 0 1 30

default 10.10.3.126

network 10.10.3.0 255.255.255.128

exit

!

ip dhcp pool F-Security

dns 200.100.0.20

lease 0 1 30

default 10.40.3.254

network 10.40.3.0 255.255.255.0

exit

!

ip dhcp pool F-Vezetoseg

dns 200.100.0.20

lease 0 1 30

default 10.60.3.254

network 10.60.3.0 255.255.255.0

exit

RF-2 forgalomirányító

interface gig0/0

description RF2-RF

ip address 10.100.3.2 255.255.255.252

no shutdown

exit

!

interface gig0/2

description RF2-SwF

ip address 10.100.3.6 255.255.255.252

no shutdown

exit

!

interface gig0/0/0

description WAN

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

!

interface lo0

description Router-ID

ip address 10.200.3.2 255.255.255.255

no shutdown

exit

interface gig0/1

description F-WiFi-GW

ip address 10.10.3.254 255.255.255.128

no shutdown

exit

!

ip dhcp excluded-address 10.10.3.254

!

ip dhcp pool F-WiFi

dns 10.80.0.2

lease 0 1 30

default 10.10.3.254

network 10.10.3.128 255.255.255.128

exit

SW F kapcsoló

vlan 10

name F-Dolgozok

vlan 40

name F-Security

vlan 60

name F-Vezetoseg

exit

!

interface vlan 1

description Mngmnt

ip address 10.100.3.9 255.255.255.252

no shutdown

SW F-2 kapcsoló

vlan 40

name F-Security

vlan 60

name F-Vezetoseg

exit

!

interface vlan 1

description Mngmnt

ip address 10.100.3.10 255.255.255.252

no shutdown

Kávézó épület

RK forgalomirányító

int gig0/0

desc RK-APK

ip address 10.10.4.254 255.255.255.0

no sh

exit

!

int gig0/1

desc RK-SwK

ip address 10.100.4.1 255.255.255.0

no sh

exit

!

interface gig0/1.60

description R-Vezetoseg-GW

encapsulation dot1q 60

ip address 10.60.4.254 255.255.255.0

exit

!

int gig0/2

desc RK-APKG

ip address 10.20.4.254 255.255.255.0

no sh

exit

!

int gig0/0/0

desc ISP

ip address 200.100.0.7 255.255.255.252

no sh

exit

!

ip dhcp excluded-address 10.20.4.254

ip dhcp excluded-address 10.10.4.254

ip dhcp excluded-address 10.60.4.254

!

ip dhcp pool K-Vezetoseg

dns 200.100.0.20

lease 0 1 30

default 10.60.4.254

network 10.60.4.0 255.255.255.0

!

ip dhcp pool K-Dolgozok

dns 200.100.0.20

lease 0 1 30

default 10.10.4.254

network 10.10.4.0 255.255.255.0

!

ip dhcp pool K-Vendeg

dns 200.100.0.20

lease 0 1 30

default 10.20.4.254

network 10.20.4.0 255.255.255.0

!

interface lo0

desc Router-ID

ip address 10.200.4.1 255.255.255.0

no sh

exit

SW K kapcsoló

vlan 60

name K-Vezetoseg

exit

!

interface vlan 1

description Mngmnt

ip address 10.100.4.2 255.255.255.0

IPv6 címek konfigurációja

Kávézó épület

RK forgalomirányító

ipv6 unicast-routing

!

ipv6 dhcp pool LAN

address prefix 2001:db8:1984:604::/64

exit

!

interface gig0/1.60

ipv6 enable

ipv6 address prefix 2001:db8:1984:604::1/64

ipv6 dhcp server LAN

ipv6 nd managed-config-flag

exit

Második rétegbeli redundancia megvalósítása

Vezetőség épület

6. ábra: Második rétegbeli redundancia a Vezetőség épületben

SW V kapcsoló

spanning-tree vlan 30,40,50,70,100

spanning-tree mode rapid-pvst

!

interface range fa0/21 - 22

description SwV-SwV3

channel-group 3 mode active

channel-protocol lacp

exit

!

interface range fa0/23 - 24

description SwV-SwV2

channel-group 1 mode active

channel-protocol lacp

exit

!

interface po1

switchport mode trunk

switchport trunk allowed vlan 30,40,50,70,100

switchport trunk native vlan 99

exit

!

interface po3

switchport mode trunk

switchport trunk allowed vlan 30,40,50,70,100

switchport trunk native vlan 99

exit

SW V-2 kapcsoló

spanning-tree vlan 30,40,50,70,100

spanning-tree mode rapid-pvst

!

interface range fa0/21 - 22

description SwV-SwV2

channel-group 2 mode active

channel-protocol lacp

exit

!

interface range fa0/23 - 24

description SwV-SwV2

channel-group 1 mode active

channel-protocol lacp

exit

!

interface po1

switchport mode trunk

switchport trunk allowed vlan 30,40,50,70,100

switchport trunk native vlan 99

exit

!

interface po2

switchport mode trunk

switchport trunk allowed vlan 30,40,50,70,100

switchport trunk native vlan 99

exit

SW V-3 kapcsoló

spanning-tree vlan 30,40,50,70,100

spanning-tree mode rapid-pvst

!

interface range fa0/21 - 22

description SwV-SwV3

channel-group 3 mode active

channel-protocol lacp

exit

!

interface range fa0/23 - 24

description SwV2-SwV3

channel-group 2 mode active

channel-protocol lacp

exit

!

interface po2

switchport mode trunk

switchport trunk allowed vlan 30,40,50,70,100

switchport trunk native vlan 99

exit

!

interface po3

switchport mode trunk

switchport trunk allowed vlan 30,40,50,70,100

switchport trunk native vlan 99

exit

Fejlesztés épület

7. ábra: Második rétegbeli redundancia a Fejlesztés épületben

SW F kapcsoló

interface range fa0/23 - 24

description SwF-SwF-2

channel-group 1 mode active

channel-protocol lacp

exit

!

interface po1

switchport mode trunk

switchport trunk allowed vlan 10,40,60,100

switchport trunk native vlan 99

exit

SW F-2 kapcsoló

interface range fa0/23 - 24

description SwF-SwF-2

channel-group 1 mode active

channel-protocol lacp

exit

!

interface po1

switchport mode trunk

switchport trunk allowed vlan 10,40,60,100

switchport trunk native vlan 99

exit

Harmadik rétegbeli redundancia megvalósítása

Vezetőség épület

8. ábra: Harmadik rétegbeli redundancia a Vezetőség épületben

RV forgalomirányító

interface g0/1

standby version 2

standby 1 ip 10.100.1.254

standby 1 priority 150

standby 1 preempt

exit

!

interface gig0/1.30

standby version 2

standby 1 ip 10.30.1.254

standby 1 priority 150

standby 1 preempt

exit

!

interface gig0/1.40

standby version 2

standby 1 ip 10.40.1.254

standby 1 priority 150

standby 1 preempt

exit

!

interface gig0/1.50

standby version 2

standby 1 ip 10.50.1.254

standby 1 priority 150

standby 1 preempt

exit

!

interface gig0/1.70

standby version 2

standby 1 ip 10.70.1.254

standby 1 priority 150

standby 1 preempt

exit

!

interface gig0/1.80

standby version 2

standby 1 ip 10.80.1.254

standby 1 priority 150

standby 1 preempt

exit

RV-2 forgalomirányító

interface g0/1

standby version 2

standby 1 ip 10.100.1.254

exit

!

interface gig0/1.30

standby version 2

standby 1 ip 10.30.1.254

exit

!

interface gig0/1.40

standby version 2

standby 1 ip 10.40.1.254

exit

!

interface gig0/1.50

standby version 2

standby 1 ip 10.50.1.254

exit

!

interface gig0/1.70

standby version 2

standby 1 ip 10.70.1.254

exit

!

interface gig0/1.80

standby version 2

standby 1 ip 10.80.1.254

exit

Vezetéknélküli hálózat konfigurációja

Raktár épület

9. ábra: Vezeték nélküli internetelérés topológiája a Raktár épületben

interface gig0/0

description WiFi-GW

ip address 10.10.2.254 255.255.255.0

ip nat inside

no shutdown

exit

!

ip dhcp excluded-address 10.10.2.254

!

ip dhcp pool R-WiFi

dns 200.100.0.20

lease 0 1 30

default 10.10.2.254

network 10.10.2.0 255.255.255.0

exit

Fejlesztés épület

10. ábra: Vezeték nélküli internetelérés topológiája a fejlesztés épületben

interface gig0/1

description F-WiFi-GW

ip address 10.10.3.254 255.255.255.128

no shutdown

exit

!

ip dhcp excluded-address 10.10.3.254

!

ip dhcp pool F-WiFi

dns 10.80.0.2

lease 0 1 30

default 10.10.3.254

network 10.10.3.128 255.255.255.128

exit

Kávézó épület

11. ábra: Vezeték nélküli internetelérés topológiája a kávézó épületben

int gig0/0

desc RK-APK

ip address 10.10.4.254 255.255.255.0

no sh

ip nat inside

exit

!

int gig0/2

desc RK-APKG

ip address 10.20.4.254 255.255.255.0

no sh

ip nat inside

exit

!

ip dhcp excluded-address 10.20.4.254

ip dhcp excluded-address 10.10.4.254

!

ip dhcp pool K-Dolgozok

dns 200.100.0.20

lease 0 1 30

default 10.10.4.254

network 10.10.4.0 255.255.255.0

exit

!

ip dhcp pool K-Vendeg

dns 200.100.0.20

lease 0 1 30

default 10.20.4.254

network 10.20.4.0 255.255.255.0

exit

12. ábra: AP-K konfigurációja

13. ábra: AP-KG konfigurációja

Statikus forgalomirányítás konfigurációja.

Vezetőség épület

RV forgalomirányító

ip route 0.0.0.0 0.0.0.0 200.100.0.17

Raktár épület

RR forgalomirányító

ip route 0.0.0.0 0.0.0.0 200.100.0.2

Fejlesztés épület

ASA tűzfal

route outside 0.0.0.0 0.0.0.0 200.100.0.10 1

RF forgalomirányító

ip route 0.0.0.0 0.0.0.0 10.0.0.2

Kávézó épület

RK forgalomirányító

ip route 0.0.0.0 0.0.0.0 200.100.0.6

Dinamikus forgalomirányítás konfigurációja

Fejlesztés épület

ASA tűzfal

router ospf 1

log-adjacency-changes

network 10.201.0.0 255.255.255.252 area 0

default-information originate

RF forgalomirányító

interface GigabitEthernet0/0

description RF-RF2

exit

!

router ospf 1

log-adjacency-changes

passive-interface GigabitEthernet0/1

network 10.10.3.0 0.0.0.127 area 0

network 10.40.3.0 0.0.0.255 area 0

network 10.60.3.0 0.0.0.255 area 0

network 10.201.0.0 0.0.0.3 area 0

exit

RF-2 forgalomirányító

interface GigabitEthernet0/0

description RF2-RF

exit

!

router ospf 3

log-adjacency-changes

passive-interface GigabitEthernet0/1

passive-interface GigabitEthernet0/2

network 10.10.3.128 0.0.0.127 area 3

network 10.0.0.0 0.0.0.3 area 3

network 10.100.3.0 0.0.0.7 area 3

network 10.100.3.8 0.0.0.3 area 3

exit

Statikus címfordítás megvalósítása

Vezetőség épület

RV forgalomirányító

ip nat inside source static 10.80.1.1 200.100.0.19

ip nat inside source static 10.80.1.2 200.100.0.20

RV-2 forgalomirányító

ip nat inside source static 10.80.1.1 200.100.0.19

ip nat inside source static 10.80.1.2 200.100.0.20

Dinamikus címfordítás megvalósítása

Vezetőség épület

RV forgalomirányító

ip nat inside source list LAN interface gig0/0/0 overload

!

interface gig0/0/1

description RV-SwV

ip nat inside

no shutdown

exit

!

interface gig0/0/1.30

ip nat inside

no shutdown

exit

!

interface gig0/0/1.40

ip nat inside

no shutdown

exit

!

interface gig0/0/1.50

ip nat inside

no shutdown

exit

!

interface gig0/0/1.70

ip nat inside

no shutdown

exit

!

interface gig0/0/1.80

ip nat inside

no shutdown

exit

!

interface gig0/0/2

desc ISP

ip nat outside

no shutdown

exit

Raktár épület

RR forgalomirányító

ip nat inside source list LAN interface gig0/0/0 overload

!

interface gig0/1

description RR-SwR

ip nat inside

no shutdown

exit

!

interface gig0/1.60

description R-Vezetoseg-GW

ip nat inside

no shutdown

exit

!

interface gig0/1.40

description R-Security-GW

ip nat inside

no shutdown

exit

!

interface gig0/0

description WiFi-GW

ip nat inside

no shutdown

exit

!

interface gig0/0/0

description ISP

ip nat outside

no shutdown

exit

Fejlesztés épület

ASA tűzfal

interface gig1/1

nameif inside

exit

!

interface gig1/2

nameif outside

exit

!

object network INSIDE-LEAD

subnet 10.60.3.0 255.255.255.0

nat (inside,outside) dynamic interface

object network INSIDE-SECURITY

subnet 10.40.3.0 255.255.255.0

nat (inside,outside) dynamic interface

object network INSIDE-WIFI

subnet 10.10.3.128 255.255.255.128

nat (inside,outside) dynamic interface

object network INSIDE-WORKER

subnet 10.10.3.0 255.255.255.128

nat (inside,outside) dynamic interface

exit

!

Kávézó épület

RK forgalomirányító

ip nat inside source list LAN interface gig0/0/0 overload

!

int gig0/0

desc RK-APK

ip nat inside

no shutdown

exit

!

int gig0/1

desc RK-SwK

ip nat inside

no shutdown

exit

!

interface gig0/1.60

description R-Vezetoseg-GW

ip nat inside

no shutdown

exit

!

int gig0/2

desc RK-APKG

ip nat inside

no shutdown

exit

!

int gig0/0/0

desc ISP

ip nat outside

no shutdown

exit

!

WAN összeköttetés

14. ábra: FTTB összeköttetés a Fejlesztés és a Vezetőség épületek között

Vezetőség épület

RV-2 forgalomirányító

interface GigabitEthernet0/0/0

description WAN

ip address 10.0.0.1 255.255.255.252

!

Fejlesztés épület

RF-2 forgalomirányító

interface GigabitEthernet0/0/0

description WAN

ip address 11.0.0.2 255.255.255.252

!

VPN kapcsolat megvalósítása

Vezetőség épület

15. ábra: VPN elérés az RV forgalomirányító és Home User között

RV forgalomirányító

aaa new-model

aaa authentication login VPN-user local

aaa authorization network VPN-group local

ip local pool VPN-pool 192.168.0.129 192.168.0.253

!

crypto isakmp enable

crypto isakmp policy 1

authentication pre-share

encryption aes

hash sha

group 5

lifetime 86400

exit

!

crypto isakmp client configuration group VPN-group

key cisco

pool VPN-pool

netmask 255.255.255.0

exit

!

crypto ipsec transform-set 1 esp-des esp-sha-hmac

crypto dynamic-map DMAP 1

set transform-set 1

reverse-route

exit

!

crypto map SMAP client authentication list VPN-user

crypto map SMAP isakmp authorization list VPN-group

crypto map SMAP client configuration address respond

crypto map SMAP 1 ipsec-isakmp dynamic DMAP

!

interface gig0/0/2

crypto map SMAP

exit

Hálózatkonfigurációs program

16. ábra: [Options] → [Preferences] → [Miscellaneous] fülnél bekapcsoljuk a külső elérést

17. ábra: Hálózati Kontroller beépítése a hálózatba

18. ábra: IP cím megadása a Hálózati Kontrollernek

19. ábra: Alapértelmezett átjáró beállítása a Hálózati Kontrolleren

20. ábra: Port megnyitása LAN-on

21. ábra: Elérés tesztelése külső böngészővel

22. ábra: Felhasználó létrehozása a Hálózai Kontrollerhez

23. ábra: Belépés a létrehozott felhasználóba

24. ábra: Hálózati Kontroller Webes felületének főoldala

25. ábra: SwR "enable" belépésének adatai

26. ábra: SwR hálózati eszközkénti hozzáadása

27. ábra: SwR hozzáadva hálózati eszközként és elérhető

28. ábra: SwR kapcsolón keresztül elérhető végponti eszközök és adataik

29. ábra: Hálózatkonfigurációs Python program

30. ábra: Python program kimenete

ACL-ek

Vezetőség épület

RV forgalomirányító

ip inspect dns-timeout 7

ip inspect udp idle-time 1800

ip inspect tcp idle-time 14400

ip inspect name INSPECT udp timeout 15

ip inspect name INSPECT tcp timeout 3600

interface gig0/0/2

ip inspect INSPECT out

exit

!

ip access-list extended OUTGOING

permit tcp any gt 1024 host 200.100.0.20 eq 53

permit udp any gt 1024 host 200.100.0.20 eq 80

permit udp any gt 1024 host 200.100.0.20 eq 443

!

ip access-list extended INCOMING

permit tcp host 200.100.0.20 eq 53 any gt 1024

permit udp host 200.100.0.20 eq 80 any gt 1024

permit udp host 200.100.0.20 eq 443 any gt 1024

exit

!

interface gig0/0/1

ip access-group INCOMING in

ip access-group OUTGOING out

exit

ASA tűzfal konfigurációja

Fejlesztés épület

ASA tűzfal

interface gig1/1

nameif inside

security-level 100

ip address 10.0.0.2 255.0.0.0

no shutdown

!

interface gig1/2

nameif outside

security-level 0

ip address 200.100.0.9 255.255.255.252

no shutdown

exit

!

route outside 0.0.0.0 0.0.0.0 200.100.0.10 1

!

router ospf 1

log-adjacency-changes

network 10.0.0.0 255.0.0.0 area 0

default-information originate

exit

!

object network LAN

subnet 10.0.0.0 255.0.0.0

nat (inside,outside) dynamic interface

exit

!

configure terminal

class-map INSPECTION-DEFAULT

match default-inspection-traffic

exit

policy-map GLOBAL-POLICY

class INSPECTION-DEFAULT

inspect icmp

inspect http

inspect dns

exit

service-policy GLOBAL-POLICY global

NTP és Syslog konfigurációja

Vezetés épület

Linux szerver

31. ábra: NTP konfigurációja a Linux szerveren

32. ábra: Syslog konfigurációja a Linux szerveren

RV forgalomirányító

ntp server 10.80.1.2

ntp authenticate

ntp authentication-key 1 md5 cisco

ntp update-calendar

service timestamps log datetime msec

!

logging 10.80.1.2

logging trap debugging

logging userinfo

33. ábra: NTP ellenőrzése RV forgalomirányítón

Hálózati eszközök teljes konfigurációja

Internetszolgáltató

Internet felhő

!internet

!

enable

configure terminal

hostname ISP

no ip domain-lookup

!

interface gig0/0

description Vezetes

ip address 200.100.0.17 255.255.255.248

no shutdown

!

interface gig1/0

description Raktar

ip address 200.100.0.2 255.255.255.252

no shutdown

!

interface gig2/0

description VPN-user

ip address 200.101.0.1 255.255.255.252

no shutdown

!

interface gig3/0

description Kavezo

ip address 200.100.0.6 255.255.255.252

no shutdown

!

interface gig1/0

description Fejlesztes

ip address 200.100.0.10 255.255.255.252

no shutdown

exit

!

ip route 200.100.0.19 255.255.255.255 200.100.0.18

ip route 200.100.0.20 255.255.255.255 200.100.0.18

!

end

wr

Vezetőség épület

RV forgalomirányító

!RV

!

enable

configure terminal

!

hostname RV

!

interface gig0/0/1

description RV-SwV

ip address 10.100.1.253 255.255.255.0

ip helper-address 10.80.1.2

ip nat inside

no shutdown

exit

!

interface gig0/0/2

description ISP

ip address 200.100.0.18 255.255.255.248

ip nat outside

no shutdown

exit

!

interface lo0

description Router-ID

ip address 10.200.1.1 255.255.255.255

no shutdown

exit

!

interface gig0/0/1.30

description V-Irodistak-GW

encapsulation dot1q 30

ip address 10.30.1.253 255.255.255.0

ip helper-address 10.80.1.2

ip nat inside

no shutdown

exit

!

interface gig0/0/1.40

description V-Security-GW

encapsulation dot1q 40

ip address 10.40.1.253 255.255.255.0

ip helper-address 10.80.1.2

ip nat inside

no shutdown

exit

!

interface gig0/0/1.50

description V-CEO-GW

encapsulation dot1q 50

ip address 10.50.1.253 255.255.255.0

ip helper-address 10.80.1.2

ip nat inside

no shutdown

exit

!

interface gig0/0/1.70

description V-Admin-GW

encapsulation dot1q 70

ip address 10.70.1.253 255.255.255.0

ip helper-address 10.80.1.2

ip nat inside

no shutdown

exit

!

interface gig0/0/1.80

description V-SRV-GW

encapsulation dot1q 80

ip address 10.80.1.253 255.255.255.0

ip helper-address 10.80.1.2

ip nat inside

no shutdown

exit

!

ip route 0.0.0.0 0.0.0.0 200.100.0.17

!

ip nat inside source static 10.80.1.1 200.100.0.19

ip nat inside source static 10.80.1.2 200.100.0.20

ip nat inside source list LAN interface gig0/0/2 overload

ip access-list standard LAN

remark NAT translation list

permit 10.30.1.0 0.0.0.255

permit 10.40.1.0 0.0.0.255

permit 10.50.1.0 0.0.0.255

permit 100.100.1.0 0.0.0.255

deny any

exit

!

!

!

aaa new-model

!

ntp server 10.80.1.2

ntp authenticate

ntp authentication-key 1 md5 cisco

ntp update-calendar

service timestamps log datetime msec

!

logging 10.80.1.2

logging trap debugging

logging userinfo

!

aaa authentication login LOGIN group radius

aaa authentication enable default group radius

aaa authentication login VPN-user local

aaa authorization network VPN-group local

ip local pool VPN-pool 192.168.0.129 192.168.0.253

!

crypto isakmp enable

crypto isakmp policy 1

authentication pre-share

encryption aes

hash sha

group 5

lifetime 86400

exit

!

crypto isakmp client configuration group VPN-group

key cisco

pool VPN-pool

netmask 255.255.255.0

exit

!

crypto ipsec transform-set 1 esp-des esp-sha-hmac

crypto dynamic-map DMAP 1

set transform-set 1

reverse-route

exit

!

crypto map SMAP client authentication list VPN-user

crypto map SMAP isakmp authorization list VPN-group

crypto map SMAP client configuration address respond

crypto map SMAP 1 ipsec-isakmp dynamic DMAP

!

interface gig0/0/2

crypto map SMAP

exit

!

!

! [DEBUG] ip inspect audit-trail

ip inspect dns-timeout 7

ip inspect udp idle-time 1800

ip inspect tcp idle-time 14400

ip inspect name INSPECT udp timeout 15

ip inspect name INSPECT tcp timeout 3600

interface gig0/0/2

ip inspect INSPECT out

exit

radius server LINUX

address ipv4 10.80.1.2

key cisco

exit

!

!

ip access-list extended OUTGOING

permit tcp any gt 1024 host 200.100.0.20 eq 53

permit udp any gt 1024 host 200.100.0.20 eq 80

permit udp any gt 1024 host 200.100.0.20 eq 443

deny ip any any

ip access-list extended INCOMING

permit tcp host 200.100.0.20 eq 53 any gt 1024

permit udp host 200.100.0.20 eq 80 any gt 1024

permit udp host 200.100.0.20 eq 443 any gt 1024

deny ip any any

exit

!

interface gig0/0/1

ip access-group INCOMING in

ip access-group OUTGOING out

exit

!

!

user admin privilege 15 secret cisco

user user privilege 1 secret cisco

service password-encryption

enable secret cisco

!

ip domain-name aromas-cubanos.lan

crypto key generate rsa general-keys modulus 4096

no ip domain-lookup

!

banner motd "

_                                     ____      _

/ \   _ __ ___  _ __ ___   __ _ ___   / ___|   _| |__   __ _ _ __   ___  ___

/ _ \ | '__/ _ \| '_ ` _ \ / _` / __| | |  | | | | '_ \ / _` | '_ \ / _ \/ __|

/ ___ \| | | (_) | | | | | | (_| \__ \ | |__| |_| | |_) | (_| | | | | (_) \__ \

/_/   \_\_|  \___/|_| |_| |_|\__,_|___/  \____\__,_|_.__/ \__,_|_| |_|\___/|___/

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED

You must have explicit, authorized permission to access or configure this device. Unauthorized attempts and actions to access or use this system may result in civil and/or criminal penalties.

All activities performed on this device are logged and monitored."

!

line vty 0 4

login authentication LOGIN

transport input ssh

logging synchronous

exec-timeout 10

motd

!

line vty 5 15

no login

!

line console 0

login local

logging synchronous

exec-timeout 10

motd

exit

!

interface g0/0/1

standby version 2

standby 1 ip 10.100.1.254

standby 1 priority 150

standby 1 preempt

exit

!

interface gig0/0/1.30

standby version 2

standby 1 ip 10.30.1.254

standby 1 priority 150

standby 1 preempt

exit

!

interface gig0/0/1.40

standby version 2

standby 1 ip 10.40.1.254

standby 1 priority 150

standby 1 preempt

exit

!

interface gig0/0/1.50

standby version 2

standby 1 ip 10.50.1.254

standby 1 priority 150

standby 1 preempt

exit

!

interface gig0/0/1.70

standby version 2

standby 1 ip 10.70.1.254

standby 1 priority 150

standby 1 preempt

exit

!

interface gig0/0/1.80

standby version 2

standby 1 ip 10.80.1.254

standby 1 priority 150

standby 1 preempt

exit

!

end

wr

RV-2 forgalomirányító

!RV-2

!

enable

configure terminal

!

hostname RV-2

!

interface gig0/1

description RV2-SwV3

ip address 10.100.1.5 255.255.255.0

ip helper-address 10.80.1.2

no shutdown

exit

!

interface gig0/0/0

description WAN

ip address 11.0.0.1 255.255.255.252

no shutdown

exit

!

interface lo0

description Router-ID

ip address 10.200.1.2 255.255.255.248

no shutdown

exit

!

interface gig0/1.30

description V-Irodistak-GW

encapsulation dot1q 30

ip address 10.30.1.252 255.255.255.0

ip helper-address 10.80.1.2

ip nat inside

no shutdown

exit

!

interface gig0/1.40

description V-Security-GW

encapsulation dot1q 40

ip address 10.40.1.252 255.255.255.0

ip helper-address 10.80.1.2

ip nat inside

no shutdown

exit

!

interface gig0/1.50

description V-CEO-GW

encapsulation dot1q 50

ip address 10.50.1.252 255.255.255.0

ip helper-address 10.80.1.2

ip nat inside

no shutdown

exit

!

interface gig0/1.70

description V-Admin-GW

encapsulation dot1q 70

ip address 10.70.1.252 255.255.255.0

ip helper-address 10.80.1.2

ip nat inside

no shutdown

exit

!

interface gig0/1.80

description V-SRV-GW

encapsulation dot1q 80

ip address 10.80.1.252 255.255.255.0

ip helper-address 10.80.1.2

ip nat inside

no shutdown

exit

!

!

!

!

!

!

!

user admin privilege 15 secret cisco

user user privilege 1 secret cisco

service password

enable secret cisco

!

ip domain-name aromas-cubanos.lan

crypto key generate rsa general-keys modulus 4096

no ip domain-lookup

!

banner motd "

_                                     ____      _

/ \   _ __ ___  _ __ ___   __ _ ___   / ___|   _| |__   __ _ _ __   ___  ___

/ _ \ | '__/ _ \| '_ ` _ \ / _` / __| | |  | | | | '_ \ / _` | '_ \ / _ \/ __|

/ ___ \| | | (_) | | | | | | (_| \__ \ | |__| |_| | |_) | (_| | | | | (_) \__ \

/_/   \_\_|  \___/|_| |_| |_|\__,_|___/  \____\__,_|_.__/ \__,_|_| |_|\___/|___/

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED

You must have explicit, authorized permission to access or configure this device. Unauthorized attempts and actions to access or use this system may result in civil and/or criminal penalties.

All activities performed on this device are logged and monitored."

!

line vty 0 4

login local

transport input ssh

logging synchronous

exec-timeout 10

motd

!

line vty 5 15

no login

!

line console 0

login local

logging synchronous

exec-timeout 10

motd

exit

!

interface g0/1

standby version 2

standby 1 ip 10.100.1.254

exit

!

interface gig0/1.30

standby version 2

standby 1 ip 10.30.1.254

exit

!

interface gig0/1.40

standby version 2

standby 1 ip 10.40.1.254

exit

!

interface gig0/1.50

standby version 2

standby 1 ip 10.50.1.254

exit

!

interface gig0/1.70

standby version 2

standby 1 ip 10.70.1.254

exit

!

interface gig0/1.80

standby version 2

standby 1 ip 10.80.1.254

exit

!

end

wr

SwV kapcsoló

!SwV

!

enable

configure terminal

!

hostname SwV

!

vlan 30

name V-Irodistak

vlan 40

name V-Security

vlan 50

name V-CEO

vlan 70

name V-Rendszergazda

vlan 80

name V-SERVER

vlan 100

name V-Mngmnt

exit

!

spanning-tree vlan 1,30,40,50,70,80,100

!

interface range fa0/1 - 3

description F-Irodistak

switchport mode access

switchport access vlan 30

spanning-tree portfast

spanning-tree bpdu enable

exit

!

interface gig0/1

description SwV-RV

switchport mode trunk

switchport trunk allowed vlan 1,30,40,50,70,80,100

spanning-tree portfast

spanning-tree bpdu enable

!

interface range fa0/21 - 22

description SwV-SwV3

switchport mode trunk

switchport trunk allowed vlan 1,30,40,50,70,80,100

switchport trunk native vlan 99

channel-group 3 mode active

channel-protocol lacp

exit

!

interface range fa0/23 - 24

description SwV-SwV2

switchport mode trunk

switchport trunk allowed vlan 1,30,40,50,70,80,100

switchport trunk native vlan 99

channel-group 1 mode active

channel-protocol lacp

exit

!

interface vlan 1

description Mngmnt

ip address 10.100.1.1 255.255.255.0

no shutdown

!

interface range fa0/4 - 20

shutdown

exit

!

!

!

!

!

user admin privilege 15 secret cisco

user user privilege 1 secret cisco

service password

enable secret cisco

!

ip domain-name aromas-cubanos.lan

crypto key generate rsa general-keys modulus 4096

no ip domain-lookup

ip default-gateway 10.100.1.254

!

banner motd "

_                                     ____      _

/ \   _ __ ___  _ __ ___   __ _ ___   / ___|   _| |__   __ _ _ __   ___  ___

/ _ \ | '__/ _ \| '_ ` _ \ / _` / __| | |  | | | | '_ \ / _` | '_ \ / _ \/ __|

/ ___ \| | | (_) | | | | | | (_| \__ \ | |__| |_| | |_) | (_| | | | | (_) \__ \

/_/   \_\_|  \___/|_| |_| |_|\__,_|___/  \____\__,_|_.__/ \__,_|_| |_|\___/|___/

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED

You must have explicit, authorized permission to access or configure this device. Unauthorized attempts and actions to access or use this system may result in civil and/or criminal penalties.

All activities performed on this device are logged and monitored."

!

line vty 0 4

login local

transport input ssh

logging synchronous

exec-timeout 10

motd

!

line vty 5 15

no login

!

line console 0

login local

logging synchronous

exec-timeout 10

motd

exit

!

end

wr

SwV-2 kapcsoló

!SwV-2

!

enable

configure terminal

!

hostname SwV-2

!

vlan 30

name V-Irodistak

vlan 40

name V-Security

vlan 50

name V-CEO

vlan 70

name V-Rendszergazda

vlan 80

name V-SERVER

vlan 100

name V-Mngmnt

exit

!

spanning-tree vlan 1,30,40,50,70,80,100

!

interface fa0/1

description V-Security

switchport mode access

switchport access vlan 40

spanning-tree portfast

spanning-tree bpdu enable

exit

!

interface fa0/2

description V-CEO

switchport mode access

switchport access vlan 50

spanning-tree portfast

spanning-tree bpdu enable

exit

!

interface range fa0/21 - 22

description SwV-SwV3

switchport mode trunk

switchport trunk allowed vlan 30,40,50,70,80,100

switchport trunk native vlan 99

channel-group 2 mode active

channel-protocol lacp

switchport trunk native vlan 99

exit

!

interface range fa0/23 - 24

description SwV-SwV2

switchport mode trunk

switchport trunk allowed vlan 30,40,50,70,80,100

switchport trunk native vlan 99

channel-group 1 mode active

channel-protocol lacp

switchport trunk native vlan 99

exit

!

interface vlan 1

description Mngmnt

ip address 10.100.1.2 255.255.255.0

no shutdown

!

interface range fa0/4 - 20

shutdown

exit

!

interface r gig0/1 - 2

shutdown

exit

!

!

!

!

!

user admin privilege 15 secret cisco

user user privilege 1 secret cisco

service password

enable secret cisco

!

ip domain-name aromas-cubanos.lan

crypto key generate rsa general-keys modulus 4096

no ip domain-lookup

ip default-gateway 10.100.1.254

!

banner motd "

_                                     ____      _

/ \   _ __ ___  _ __ ___   __ _ ___   / ___|   _| |__   __ _ _ __   ___  ___

/ _ \ | '__/ _ \| '_ ` _ \ / _` / __| | |  | | | | '_ \ / _` | '_ \ / _ \/ __|

/ ___ \| | | (_) | | | | | | (_| \__ \ | |__| |_| | |_) | (_| | | | | (_) \__ \

/_/   \_\_|  \___/|_| |_| |_|\__,_|___/  \____\__,_|_.__/ \__,_|_| |_|\___/|___/

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED

You must have explicit, authorized permission to access or configure this device. Unauthorized attempts and actions to access or use this system may result in civil and/or criminal penalties.

All activities performed on this device are logged and monitored."

!

line vty 0 4

login local

transport input ssh

logging synchronous

exec-timeout 10

motd

!

line vty 5 15

no login

!

line console 0

login local

logging synchronous

exec-timeout 10

motd

exit

!

end

wr

SwV-3 kapcsoló

!SwV-3

!

enable

configure terminal

!

hostname SwV-3

!

vlan 30

name V-Irodistak

vlan 40

name V-Security

vlan 50

name V-CEO

vlan 70

name V-Rendszergazda

vlan 80

name V-SERVER

vlan 100

name V-Mngmnt

exit

!

spanning-tree vlan 1,30,40,50,70,80,100

!

interface range fa0/1 - 2

description V-Rendszergazda

switchport mode access

switchport access vlan 70

spanning-tree bpdu enable

spanning-tree portfast

exit

!

interface range fa0/19-20

description SERVERS

switchport mode access

switchport access vlan 80

spanning-tree bpdu enable

spanning-tree portfast

no shutdown

exit

!

interface range fa0/21 - 22

description SwV-SwV3

switchport mode trunk

switchport trunk allowed vlan 1,30,40,50,70,80,100

switchport trunk native vlan 99

channel-group 3 mode active

channel-protocol lacp

exit

!

interface range fa0/23 - 24

description SwV2-SwV3

switchport mode trunk

switchport trunk allowed vlan 1,30,40,50,70,80,100

switchport trunk native vlan 99

channel-group 2 mode active

channel-protocol lacp

exit

!

interface gig0/1

description SwV3-RV2

switchport mode trunk

switchport trunk allowed vlan 1,30,40,50,70,80,100

switchport trunk native vlan 99

exit

!

interface vlan 1

description Mngmnt

ip address 10.100.1.3 255.255.255.0

no shutdown

!

interface range fa0/4 - 18

shutdown

exit

!

interface gig0/2

shutdown

exit

!

!

!

!

!

!

user admin privilege 15 secret cisco

user user privilege 1 secret cisco

service password

enable secret cisco

!

ip domain-name aromas-cubanos.lan

crypto key generate rsa general-keys modulus 4096

no ip domain-lookup

ip default-gateway 10.100.1.254

!

banner motd "

_                                     ____      _

/ \   _ __ ___  _ __ ___   __ _ ___   / ___|   _| |__   __ _ _ __   ___  ___

/ _ \ | '__/ _ \| '_ ` _ \ / _` / __| | |  | | | | '_ \ / _` | '_ \ / _ \/ __|

/ ___ \| | | (_) | | | | | | (_| \__ \ | |__| |_| | |_) | (_| | | | | (_) \__ \

/_/   \_\_|  \___/|_| |_| |_|\__,_|___/  \____\__,_|_.__/ \__,_|_| |_|\___/|___/

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED

You must have explicit, authorized permission to access or configure this device. Unauthorized attempts and actions to access or use this system may result in civil and/or criminal penalties.

All activities performed on this device are logged and monitored."

!

line vty 0 4

login local

transport input ssh

logging synchronous

exec-timeout 10

motd

!

line vty 5 15

no login

!

line console 0

login local

logging synchronous

exec-timeout 10

motd

exit

!

end

wr

Raktár épület

RR forgalomirányító

!RR

!

enable

configure terminal

!

hostname RR

!

interface gig0/1

description RR-SwR

ip address 192.168.0.254 255.255.255.0

no shutdown

ip nat inside

exit

!

interface gig0/1.60

description R-Vezetoseg-GW

encapsulation dot1q 60

ip address 10.60.2.254 255.255.255.0

ip nat inside

exit

!

interface gig0/1.40

description R-Security-GW

encapsulation dot1q 40

ip address 10.40.2.254 255.255.255.0

ip nat inside

exit

!

interface gig0/1.100

description R-Mngmnt-GW

encapsulation dot1q 40

ip address 10.100.2.254 255.255.255.0

ip nat inside

exit

!

interface gig0/0/0

description ISP

ip address 200.100.0.1 255.255.255.252

ip nat outside

no shutdown

exit

!

interface gig0/0

description WiFi-GW

ip address 10.10.2.254 255.255.255.0

ip nat inside

no shutdown

exit

!

ip dhcp excluded-address 10.10.2.254

ip dhcp excluded-address 10.40.2.254

ip dhcp excluded-address 10.60.2.254

ip dhcp pool R-Vezetoseg

dns 200.100.0.20

lease 0 1 30

default 10.60.2.254

network 10.60.2.0 255.255.255.0

exit

!

ip dhcp pool R-Security

dns 200.100.0.20

lease 0 1 30

default 10.40.2.254

network 10.40.2.0 255.255.255.0

exit

!

ip dhcp pool R-WiFi

dns 200.100.0.20

lease 0 1 30

default 10.10.2.254

network 10.10.2.0 255.255.255.0

exit

!

interface lo0

description Router-ID

ip address 10.200.2.1 255.255.255.255

no shutdown

!

ip route 0.0.0.0 0.0.0.0 200.100.0.2

!

ip nat inside source list LAN interface gig0/0/0 overload

ip access-list standard LAN

remark NAT translation list

permit 10.10.2.0 0.0.0.255

permit 10.40.2.0 0.0.0.255

permit 10.60.2.0 0.0.0.255

permit 100.100.2.0 0.0.0.255

deny any

exit

!

!

!

!

!

!

!

user admin privilege 15 secret cisco

user user privilege 1 secret cisco

service password

enable secret cisco

!

ip domain-name aromas-cubanos.lan

crypto key generate rsa general-keys modulus 4096

no ip domain-lookup

!

banner motd "

_                                     ____      _

/ \   _ __ ___  _ __ ___   __ _ ___   / ___|   _| |__   __ _ _ __   ___  ___

/ _ \ | '__/ _ \| '_ ` _ \ / _` / __| | |  | | | | '_ \ / _` | '_ \ / _ \/ __|

/ ___ \| | | (_) | | | | | | (_| \__ \ | |__| |_| | |_) | (_| | | | | (_) \__ \

/_/   \_\_|  \___/|_| |_| |_|\__,_|___/  \____\__,_|_.__/ \__,_|_| |_|\___/|___/

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED

You must have explicit, authorized permission to access or configure this device. Unauthorized attempts and actions to access or use this system may result in civil and/or criminal penalties.

All activities performed on this device are logged and monitored."

!

line vty 0 4

login local

transport input ssh

logging synchronous

exec-timeout 10

motd

!

line vty 5 15

no login

!

line console 0

login local

logging synchronous

exec-timeout 10

motd

exit

!

end

wr

SwR kapcsoló

!SwR

!

enable

configure terminal

hostname SwR

!

vlan 40

name R-Security

vlan 60

name R-Vezetoseg

exit

!

interface fa0/1

description R-Vezetoseg

switchport mode access

switchport access vlan 60

exit

!

interface fa0/2

description R-Security

switchport mode access

switchport access vlan 40

exit

!

interface gig0/1

description SwR-RR

switchport mode trunk

switchport trunk allowed vlan 1,40,60,100

exit

!

interface vlan 1

description Mngmnt

ip address 10.100.2.2 255.255.255.252

!

interface range fa0/3 - 24

shutdown

exit

!

interface gig0/2

shutdown

exit

!

!

!

!

!

user admin privilege 15 secret cisco

user user privilege 1 secret cisco

service password

enable secret cisco

ip domain-name aromas-cubanos.lan

crypto key generate rsa general-keys modulus 4096

no ip domain-lookup

banner motd "

_                                     ____      _

/ \   _ __ ___  _ __ ___   __ _ ___   / ___|   _| |__   __ _ _ __   ___  ___

/ _ \ | '__/ _ \| '_ ` _ \ / _` / __| | |  | | | | '_ \ / _` | '_ \ / _ \/ __|

/ ___ \| | | (_) | | | | | | (_| \__ \ | |__| |_| | |_) | (_| | | | | (_) \__ \

/_/   \_\_|  \___/|_| |_| |_|\__,_|___/  \____\__,_|_.__/ \__,_|_| |_|\___/|___/

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED

You must have explicit, authorized permission to access or configure this device. Unauthorized attempts and actions to access or use this system may result in civil and/or criminal penalties.

All activities performed on this device are logged and monitored."

!

line vty 0 4

login local

transport input ssh

logging synchronous

exec-timeout 10

motd

!

line vty 5 15

no login

!

line console 0

login local

logging synchronous

exec-timeout 10

motd

exit

end

Fejlesztés épület

ASA tűzfal

!ASA

!

enable

configure terminal

hostname ASA

!

no service-policy global_policy global

no policy-map type inspect dns preset_dns_map

no policy-map global_policy

no class-map inspection_default

!

!

!

interface gig1/1

nameif inside

security-level 100

ip address 10.0.0.2 255.0.0.0

no shutdown

interface gig1/2

nameif outside

security-level 0

ip address 200.100.0.9 255.255.255.252

no shutdown

exit

!

route outside 0.0.0.0 0.0.0.0 200.100.0.10 1

!

router ospf 1

log-adjacency-changes

network 10.0.0.0 255.0.0.0 area 0

default-information originate

exit

!

object network LAN

subnet 10.0.0.0 255.0.0.0

nat (inside,outside) dynamic interface

exit

!

configure terminal

class-map INSPECTION-DEFAULT

match default-inspection-traffic

exit

!

policy-map GLOBAL-POLICY

class INSPECTION-DEFAULT

inspect icmp

inspect http

inspect dns

exit

service-policy GLOBAL-POLICY global

!

end

wr

RF forgalomirányító

!RF

!

enable

configure terminal

hostname RF

!

interface gig0/1

description RF-SwF

ip address 10.100.3.5 255.255.255.252

no shutdown

exit

!

interface gig0/0

description RF-RF2

ip address 10.100.3.1 255.255.255.252

no shutdown

exit

!

interface gig0/2

description RF-ASA

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

!

interface lo0

description Router-ID

ip address 10.200.3.1 255.255.255.255

no shutdown

exit

!

interface gig0/1.10

description F-Dolgozok-GW

encapsulation dot1q 10

ip address 10.10.3.126 255.255.255.128

no shutdown

exit

!

interface gig0/1.40

description F-Security-GW

encapsulation dot1q 40

ip address 10.40.3.254 255.255.255.0

no shutdown

exit

!

interface gig0/1.60

description F-Vezetoseg-GW

encapsulation dot1q 60

ip address 10.60.3.254 255.255.255.0

no shutdown

exit

!

ip dhcp excluded-address 10.10.3.126

ip dhcp excluded-address 10.40.3.254

ip dhcp excluded-address 10.60.3.254

ip dhcp pool F-Dolgozok

dns 200.100.0.20

lease 0 1 30

default 10.10.3.126

network 10.10.3.0 255.255.255.128

exit

!

ip dhcp pool F-Security

dns 200.100.0.20

lease 0 1 30

default 10.40.3.254

network 10.40.3.0 255.255.255.0

exit

!

ip dhcp pool F-Vezetoseg

dns 200.100.0.20

lease 0 1 30

default 10.60.3.254

network 10.60.3.0 255.255.255.0

exit

!

ip route 0.0.0.0 0.0.0.0 10.0.0.2

!

router ospf 1

passive-interface GigabitEthernet0/1

network 10.10.3.0 0.0.0.127 area 0

network 10.40.3.0 0.0.0.255 area 0

network 10.60.3.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.3 area 0

exit

!

!

!

!

!

!

user admin privilege 15 secret cisco

user user privilege 1 secret cisco

service password-encryption

enable secret cisco

!

ip domain-name aromas-cubanos.lan

crypto key generate rsa general-keys modulus 4096

no ip domain-lookup

!

banner motd "

_                                     ____      _

/ \   _ __ ___  _ __ ___   __ _ ___   / ___|   _| |__   __ _ _ __   ___  ___

/ _ \ | '__/ _ \| '_ ` _ \ / _` / __| | |  | | | | '_ \ / _` | '_ \ / _ \/ __|

/ ___ \| | | (_) | | | | | | (_| \__ \ | |__| |_| | |_) | (_| | | | | (_) \__ \

/_/   \_\_|  \___/|_| |_| |_|\__,_|___/  \____\__,_|_.__/ \__,_|_| |_|\___/|___/

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED

You must have explicit, authorized permission to access or configure this device. Unauthorized attempts and actions to access or use this system may result in civil and/or criminal penalties.

All activities performed on this device are logged and monitored."

!

line vty 0 4

login local

transport input ssh

logging synchronous

exec-timeout 10

motd

!

line vty 5 15

no login

!

line console 0

login local

logging synchronous

exec-timeout 10

motd

exit

!

end

wr

RF-2 forgalomirányító

!RF-2

!

enable

configure terminal

hostname RF-2

!

interface gig0/0

description RF2-RF

ip address 10.100.3.2 255.255.255.252

no shutdown

exit

!

interface gig0/2

description RF2-SwF

ip address 10.100.3.6 255.255.255.252

no shutdown

exit

!

interface gig0/0/0

description WAN

ip address 11.0.0.2 255.255.255.252

no shutdown

exit

!

interface lo0

description Router-ID

ip address 10.200.3.2 255.255.255.255

no shutdown

exit

!

interface gig0/1

description F-WiFi-GW

ip address 10.10.3.254 255.255.255.128

no shutdown

exit

!

ip dhcp excluded-address 10.10.3.254

!

ip dhcp pool F-WiFi

dns 10.80.0.2

lease 0 1 30

default 10.10.3.254

network 10.10.3.128 255.255.255.128

exit

!

router ospf 3

network 10.10.3.128 0.0.0.127 area 3

network 11.0.0.0 0.0.0.3 area 3

network 10.100.3.0 0.0.0.7 area 3

network 10.100.3.8 0.0.0.3 area 3

passive gig0/1

passive gig0/2

passive gig0/0/0

exit

!

!

!

!

!

!

!

user admin privilege 15 secret cisco

user user privilege 1 secret cisco

service password

enable secret cisco

!

ip domain-name aromas-cubanos.lan

crypto key generate rsa general-keys modulus 4096

no ip domain-lookup

!

banner motd "

_                                     ____      _

/ \   _ __ ___  _ __ ___   __ _ ___   / ___|   _| |__   __ _ _ __   ___  ___

/ _ \ | '__/ _ \| '_ ` _ \ / _` / __| | |  | | | | '_ \ / _` | '_ \ / _ \/ __|

/ ___ \| | | (_) | | | | | | (_| \__ \ | |__| |_| | |_) | (_| | | | | (_) \__ \

/_/   \_\_|  \___/|_| |_| |_|\__,_|___/  \____\__,_|_.__/ \__,_|_| |_|\___/|___/

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED

You must have explicit, authorized permission to access or configure this device. Unauthorized attempts and actions to access or use this system may result in civil and/or criminal penalties.

All activities performed on this device are logged and monitored."

!

line vty 0 4

login local

transport input ssh

logging synchronous

exec-timeout 10

motd

!

line vty 5 15

no login

!

line console 0

login local

logging synchronous

exec-timeout 10

motd

exit

!

end

wr

SwF kapcsoló

!SwF

!

enable

configure terminal

hostname SwF

!

vlan 10

name F-Dolgozok

vlan 40

name F-Security

vlan 60

name F-Vezetoseg

exit

!

interface range fa0/1 - 3

description F-Dolgozok

switchport mode access

switchport access vlan 10

exit

!

interface gig0/1

description SwF-RF

switchport mode trunk

switchport trunk allowed vlan 1,10,40,60,100

!

interface gig0/2

description SwF-RF-2

switchport mode trunk

switchport trunk allowed vlan 1,10,40,60,100

!

interface range fa0/23 - 24

description SwF-SwF-2

channel-group 1 mode active

channel-protocol lacp

exit

!

interface po1

switchport mode trunk

switchport trunk allowed vlan 1,10,40,60,100

switchport trunk native vlan 99

exit

!

interface vlan 1

description Mngmnt

ip address 10.100.3.9 255.255.255.252

no shutdown

!

interface range fa0/4 - 22

shutdown

exit

!

!

!

!

!

user admin privilege 15 secret cisco

user user privilege 1 secret cisco

service password

enable secret cisco

ip domain-name aromas-cubanos.lan

crypto key generate rsa general-keys modulus 4096

no ip domain-lookup

banner motd "

_                                     ____      _

/ \   _ __ ___  _ __ ___   __ _ ___   / ___|   _| |__   __ _ _ __   ___  ___

/ _ \ | '__/ _ \| '_ ` _ \ / _` / __| | |  | | | | '_ \ / _` | '_ \ / _ \/ __|

/ ___ \| | | (_) | | | | | | (_| \__ \ | |__| |_| | |_) | (_| | | | | (_) \__ \

/_/   \_\_|  \___/|_| |_| |_|\__,_|___/  \____\__,_|_.__/ \__,_|_| |_|\___/|___/

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED

You must have explicit, authorized permission to access or configure this device. Unauthorized attempts and actions to access or use this system may result in civil and/or criminal penalties.

All activities performed on this device are logged and monitored."

!

line vty 0 4

login local

transport input ssh

logging synchronous

exec-timeout 10

motd

!

line vty 5 15

no login

!

line console 0

login local

logging synchronous

exec-timeout 10

motd

exit

end

SwF-2 kapcsoló

!SwF-2

!

enable

configure terminal

hostname SwF-2

!

vlan 40

name F-Security

vlan 60

name F-Vezetoseg

exit

!

interface fa0/1

description F-Security

switchport mode access

switchport access vlan 40

exit

!

interface fa0/2

description F-Vezetoseg

switchport mode access

switchport access vlan 60

exit

!

interface range fa0/23 - 24

description SwF-SwF-2

channel-group 1 mode active

channel-protocol lacp

exit

!

interface po1

switchport mode trunk

switchport trunk allowed vlan 1,10,40,60,100

switchport trunk native vlan 99

exit

!

interface vlan 1

description Mngmnt

ip address 10.100.3.10 255.255.255.252

no shutdown

!

interface range fa0/4 - 22

shutdown

exit

interface range gig0/1 - 2

shutdown

exit

!

!

!

!

!

!

user admin privilege 15 secret cisco

user user privilege 1 secret cisco

service password

enable secret cisco

ip domain-name aromas-cubanos.lan

crypto key generate rsa general-keys modulus 4096

no ip domain-lookup

banner motd "

_                                     ____      _

/ \   _ __ ___  _ __ ___   __ _ ___   / ___|   _| |__   __ _ _ __   ___  ___

/ _ \ | '__/ _ \| '_ ` _ \ / _` / __| | |  | | | | '_ \ / _` | '_ \ / _ \/ __|

/ ___ \| | | (_) | | | | | | (_| \__ \ | |__| |_| | |_) | (_| | | | | (_) \__ \

/_/   \_\_|  \___/|_| |_| |_|\__,_|___/  \____\__,_|_.__/ \__,_|_| |_|\___/|___/

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED

You must have explicit, authorized permission to access or configure this device. Unauthorized attempts and actions to access or use this system may result in civil and/or criminal penalties.

All activities performed on this device are logged and monitored."

!

line vty 0 4

login local

transport input ssh

logging synchronous

exec-timeout 10

motd

!

line vty 5 15

no login

!

line console 0

login local

logging synchronous

exec-timeout 10

motd

exit

end

Kávézó épület

RK forgalomirányító

enable

configure terminal

!

hostname RK

!

int gig0/0

desc RK-APK

ip address 10.10.4.254 255.255.255.0

no sh

ip nat inside

exit

!

int gig0/1

desc RK-SwK

ip address 10.100.4.1 255.255.255.0

no sh

ip nat inside

exit

!

interface gig0/1.60

description R-Vezetoseg-GW

encapsulation dot1q 60

ip address 10.60.4.254 255.255.255.0

ip nat inside

exit

!

int gig0/2

desc RK-APKG

ip address 10.20.4.254 255.255.255.0

no sh

ip nat inside

exit

!

int gig0/0/0

desc ISP

ip address 200.100.0.7 255.255.255.252

ip nat outside

no sh

exit

!

ip dhcp excluded-address 10.20.4.254

ip dhcp excluded-address 10.10.4.254

ip dhcp excluded-address 10.60.4.254

ip dhcp pool K-Vezetoseg

dns 200.100.0.20

lease 0 1 30

default 10.60.4.254

network 10.60.4.0 255.255.255.0

exit

!

ip dhcp pool K-Dolgozok

dns 200.100.0.20

lease 0 1 30

default 10.10.4.254

network 10.10.4.0 255.255.255.0

exit

!

ip dhcp pool K-Vendeg

dns 200.100.0.20

lease 0 1 30

default 10.20.4.254

network 10.20.4.0 255.255.255.0

exit

!

interface lo0

desc Router-ID

ip address 10.200.4.1 255.255.255.0

no sh

exit

!

ip route 0.0.0.0 0.0.0.0 200.100.0.6

!

ip nat inside source list LAN interface gig0/0/0 overload

ip access-list standard LAN

remark NAT translation list

permit 10.10.4.0 0.0.0.255

permit 10.20.4.0 0.0.0.255

permit 10.60.4.0 0.0.0.255

permit 100.100.4.0 0.0.0.255

deny any

exit

!

!

!

ipv6 unicast-routing

ipv6 dhcp pool LAN

address prefix 2001:db8:1984:604::/64

exit

interface gig0/1.60

ipv6 enable

ipv6 address prefix 2001:db8:1984:604::1/64

ipv6 dhcp server LAN

ipv6 nd managed-config-flag

exit

!

!

!

!

user admin privilege 15 secret cisco

user user privilege 1 secret cisco

service password

enable secret cisco

!

ip domain-name aromas-cubanos.lan

crypto key generate rsa general-keys modulus 4096

no ip domain-lookup

!

banner motd "

_                                     ____      _

/ \   _ __ ___  _ __ ___   __ _ ___   / ___|   _| |__   __ _ _ __   ___  ___

/ _ \ | '__/ _ \| '_ ` _ \ / _` / __| | |  | | | | '_ \ / _` | '_ \ / _ \/ __|

/ ___ \| | | (_) | | | | | | (_| \__ \ | |__| |_| | |_) | (_| | | | | (_) \__ \

/_/   \_\_|  \___/|_| |_| |_|\__,_|___/  \____\__,_|_.__/ \__,_|_| |_|\___/|___/

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED

You must have explicit, authorized permission to access or configure this device. Unauthorized attempts and actions to access or use this system may result in civil and/or criminal penalties.

All activities performed on this device are logged and monitored."

!

line vty 0 4

login local

transport input ssh

logging synchronous

exec-timeout 10

motd

!

line vty 5 15

no login

!

line console 0

login local

logging synchronous

exec-timeout 10

motd

exit

!

end

wr

SwK kapcsoló

!SwK

enable

configure terminal

!

hostname SwK

!

vlan 60

name K-Vezetoseg

exit

!

interface fa0/1

desc K-Vezetoseg

switchport mode access

switchport access vlan 60

spanning-tree portfast

spanning-tree bpdu enable

exit

!

interface gig0/1

description SwK-RK

switchport mode trunk

switchport trunk allowed vlan 1,60,100

spanning-tree portfast

spanning-tree bpdu enable

!

interface vlan 1

description Mngmnt

ip address 10.100.4.2 255.255.255.0

!

interface range fa0/2 - 24

shutdown

exit

!

interface gig0/2

shutdown

exit

!

!

!

!

!

user admin privilege 15 secret cisco

user user privilege 1 secret cisco

service password

enable secret cisco

!

ip domain-name aromas-cubanos.lan

crypto key generate rsa general-keys modulus 4096

no ip domain-lookup

!

banner motd "

_                                     ____      _

/ \   _ __ ___  _ __ ___   __ _ ___   / ___|   _| |__   __ _ _ __   ___  ___

/ _ \ | '__/ _ \| '_ ` _ \ / _` / __| | |  | | | | '_ \ / _` | '_ \ / _ \/ __|

/ ___ \| | | (_) | | | | | | (_| \__ \ | |__| |_| | |_) | (_| | | | | (_) \__ \

/_/   \_\_|  \___/|_| |_| |_|\__,_|___/  \____\__,_|_.__/ \__,_|_| |_|\___/|___/

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED

You must have explicit, authorized permission to access or configure this device. Unauthorized attempts and actions to access or use this system may result in civil and/or criminal penalties.

All activities performed on this device are logged and monitored."

!

line vty 0 4

login local

transport input ssh

logging synchronous

exec-timeout 10

motd

!

line vty 5 15

no login

!

line console 0

login local

logging synchronous

exec-timeout 10

motd

exit

!

end

wr

Windows szerver telepítése virtuális gépre

Címtár (Active Directory) megvalósítása

Fájl és nyomtató megosztás konfigurálása

Automatizált szoftvertelepítés konfigurálása

Automatizált mentés konfigurálása

Linux szerver telepítése virtuális gépre

Operációs rendszer feltelepítése

34. ábra: Ubuntu felhasználó létrehozása

35. ábra: Új hálózati kártya hozzáadása

36. ábra: Az új hálózati kártya beállítása

37. ábra: Az új kártya működésének tesztelése

DHCP megvalósítása

38. ábra: Szükséges csomagok letöltése és telepítése DHCP-hez

39. ábra: DHCP konfigurációs fájl (1. rész)

40. ábra: DHCP konfigurációs fájl (2. rész)

41. ábra: Címosztás interfészhez rendelése

42. ábra: Szolgáltatás futásának ellenőrzése

DNS szerver megvalósítása

43. ábra: Jelenlegi konfiguráció leállítása és törlése

44. ábra:Szükséges csomagok letöltése és telepítése DNS-hez(1. rész)

45. ábra: Szükséges csomagok letöltése és telepítése DNS-hez(2. rész)

46. ábra: 53-as port beállítása

47. ábra: DNS szűrő beállítása

48. ábra: IP cím beállítása

49. ábra: Weboldalunk domain-jének beállítása

50. ábra: Szolgáltatás újraindítása és ellenőrzése

51. ábra: Szerver beállítása névszerverként (1. rész)

52. ábra: Szerver beállítása névszerverként (2. rész)

HTTP/HTTPS szerver konfigurálása

53. ábra: Szükséges csomagok letöltése és telepítése HTTPS-hez

54. ábra: Minimális információátadás beállítása

55. ábra: Szervernév beállítása

56. ábra: Szolgáltatás újraindítása

57. ábra: Jogosultág beállítás a mappára

58. ábra: Előre elkészített fájlok átimportálása

59. ábra: Weboldal konfigurációjának beállítása

60. ábra: Új oldal aktiválása és régi oldal deaktiválása, majd a szolgáltatás újraindítása

61. ábra: Weboldal elérés tesztelése

Cisco Packet Tracer Tesztelés

Teszteléshez használt szoftverek és/vagy parancsok

- show

- ping

- tracert

- ipconfig

- Beépített böngésző

- Beépített SSH kliens

- Beépített VPN kliens

Trunk portok

- Tesztelés megnevezése: Trunk portok státuszának tesztelése

- Elvárás: A portok státuszát írja ki a show parancs után

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

62. ábra: Trunk portok státusza

VLAN elérési teszt

- Tesztelés megnevezése: Kapcsolat 2 különböző VLAN között

- Elvárás: A csomagok megérkezzenek és visszaküldje őket a célgép

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

63. ábra: Ping Fd3 PC és Fvezetoseg PC közt

64. ábra: Adott VLAN-okhoz rendelt portok

EtherChannel teszt

- Tesztelés megnevezése: EtherChannel-en keresztül elérés SwF és SwF2 kapcsolók közt

- Elvárás: A kapcsolók elérjék egymást egyik vonal lekapcsolása esetén is

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

65. ábra: Pingelés EtherChannel-en keresztül

HSRP teszt

- Tesztelés megnevezése: HSRP tesztelése, ha az egyik forgalomirányító nem elérhető

- Elvárás: A csomagok kimenjenek a másik forgalomirányítón

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

66. ábra: HSRP tesztelése

Linux DHCP teszt

- Tesztelés megnevezése: DHCP tesztelése a Linux szerverről

- Elvárás: A Linux szerver osszon ki egy megfelelő IPv4 címet a célgépnek.

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

67. ábra: Linux IPv4 címkiosztása

DHCP teszt

- Tesztelés megnevezése: DHCP tesztelése RF forgalomirányító

- Elvárás: Az RF forgalomirányító osszon ki egy megfelelő IP címet a célgépnek.

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

68. ábra: RF forgalomirányító által kiosztott IP címek

DHCPv6 teszt

- Tesztelés megnevezése: DHCPv6 tesztelése

- Elvárás: Osszon ki egy megfelelő IPv6 címet a célgépnek.

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

69. ábra: IPv6 címkiosztása

ASA NAT teszt

- Tesztelés megnevezése: ASA tűzfal eszköz NAT teszt

- Elvárás: Az ASA tűzfal NAT-oljon

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

70. ábra: ASA NAT

Dinamikus NAT teszt

- Tesztelés megnevezése: Raktár épület forgalomirányító PAT teszt

- Elvárás: A forgalomirányító Dinamikus NAT-ot hajtson végre

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

71. ábra: RR forgalomirányító NAT

Forgalomirányítón Dinamikus és Statikus NAT teszt

- Tesztelés megnevezése: Vezetőség épület forgalomirányítójának NAT-olása

- Elvárás: A forgalomirányító NAT-oljon

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

72. ábra: RV forgalomirányító NAT

WiFi kapcsolat teszt

- Tesztelés megnevezése: AP-K tesztelése

- Elvárás: Az AP eszközre lehessen csatlakozni telefonnal internetelérés miatt

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

73. ábra: Dolgozói telefon felcsatlakozik az AP-re

Feszítőfa protokoll teszt

- Tesztelés megnevezése: DHCP tesztelése a Linux szerverről

- Elvárás: A Linux szerver osszon ki egy megfelelő címet a tesztgépnek.

- Eredmény: Sikeres a művelet.

- Bizonyíték: (kép)

74. ábra: Tesztelés előtt és közbeni kép a topológiáról

75. ábra: Feszítőfa protokoll összesítő show parancs kimenete

SSH elérés teszt

- Tesztelés megnevezése: SSH elérés Fd3 PC-ről RF forgalomirányítóra

- Elvárás: A PC be tudjon lépni SSH-val a forgalomirányítóra

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

76. ábra: SSH belépés Fd3-ról RF-re

HTTP és DNS Linux szerveren teszt

- Tesztelés megnevezése: HTTP és DNS tesztelése Fd1 PC-n

- Elvárás: A PC böngészőjében jelenjen meg a weboldal a domain alapján

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

77. ábra: HTTP weboldal elérés

HTTPS és DNS Linux szerveren teszt

- Tesztelés megnevezése: HTTPS és DNS tesztelése Fd1 PC-n

- Elvárás: A PC böngészőjében jelenjen meg a weboldal a domain alapján

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

78. ábra: HTTPS weboldal elérés

ACL teszt

- Tesztelés megnevezése: ACL RV eszköz teszt

- Elvárás: A felvett ACL-ek meg kell jelenjenek a show parancs után

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

79. ábra: ACL-ek a vezetőség elsődleges forgalomirányítón

CBAC teszt

- Tesztelés megnevezése: CBAC RV eszköz teszt

- Elvárás: A CBAC ki kell engedje a megadott üzeneteket

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

80. ábra: CBAC működése

AAA teszt

- Tesztelés megnevezése: Felhasználó kérése a RADIUS szervertől

- Elvárás: Sikeres belépés az AAA felhasználóval

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

81. ábra: Belépési adatok és sikeres belépés

Syslog teszt

- Tesztelés megnevezése: Naplózás Syslog-gal teszt

- Elvárás: Az eszköz naplózza az eseményeket

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

82. ábra: Syslog naplózás szerver szemszögből

NTP teszt

- Tesztelés megnevezése: NTP szerverrel időbeállítás

- Elvárás: Szinkronizálja az időt a szerverhez

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

83. ábra: Időszinkronizálás NTP-vel

VPN kapcsolat teszt

- Tesztelés megnevezése: VPN kapcsolat otthonról a vezetőségi épületbe

- Elvárás: A HOuser be tudjon VPN-ezni a hálózatba

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

84. ábra: VPN kapcsolat tesztelése

85. ábra: ISAKMP teszt

Hálózatkonfigurációs program teszt

- Tesztelés megnevezése: Hálózatkonfigurációs program tesztelése

- Elvárás: A Python program megmutatja melyik eszközök vannak kötve az SwR-re

- Eredmény: Sikeres a művelet.

- Bizonyíték: (kép)

86. ábra: Hálózatkonfigurációs program kimenete

Oracle VM VirtualBox tesztelés

Teszteléshez használt tesztgép operációs rendszere0

- Windows 10 2022 (22H2)

Windows szerver teszt

Linux szerver teszt

DHCP teszt

- Tesztelés megnevezése: DHCP tesztelése a Linux szerverről

- Elvárás: A Linux szerver osszon ki egy megfelelő címet a tesztgépnek.

- Eredmény: Sikeres a művelet.

- Bizonyíték: (kép)

87. ábra: Linux szerver IPv4 címet osztott ki

DNS teszt

- Tesztelés megnevezése: DNS tesztelése a Linux szerverről

- Elvárás: A Linux szerver DNS információkat küldjön

- Eredmény: Sikeres a művelet

- Bizonyíték: (kép)

88. ábra: DNS tesztelése

89. ábra: Weboldal elérése domain által

HTTP/HTTPS teszt

- Tesztelés megnevezése: Weboldal elérés tesztelése a Linux szerverről

- Elvárás: A Linux szerveren futó weboldal elérhető legyen a tesztgépről

- Eredmény: Sikeres a művelet.

- Bizonyíték: (kép)

90. ábra: Weboldal elérhető a szerver IP címe alapján

Felhasznált szoftverek

- Cisco Packet Tracer (v8.2.2)

- Github

- MS Onedrive

- MS Planner

- MS PowerPoint 2019

- MS Word 2019

- Oracle VM VirtualBox (7.0.10)

- Python 3.13.1 (v2022.16.1)

- Visual Studio Code (1.98.2)

Ábrajegyzék

1. ábra: Teljes topológia képe	6

2. ábra: Raktár topológia	6

3. ábra: Kávézó topológia	6

4. ábra: Fejlesztés topológia	7

5. ábra: Vezetőség topológia	7

6. ábra: Második rétegbeli redundancia a Vezetőség épületben	30

7. ábra: Második rétegbeli redundancia a Fejlesztés épületben	33

8. ábra: Harmadik rétegbeli redundancia a Vezetőség épületben	34

9. ábra: Vezeték nélküli internetelérés topológiája a Raktár épületben	36

10. ábra: Vezeték nélküli internetelérés topológiája a fejlesztés épületben	37

11. ábra: Vezeték nélküli internetelérés topológiája a kávézó épületben	38

12. ábra: AP-K konfigurációja	39

13. ábra: AP-KG konfigurációja	39

14. ábra: FTTB összeköttetés a Fejlesztés és a Vezetőség épületek között	47

15. ábra: VPN elérés az RV forgalomirányító és Home User között	48

16. ábra: [Options] → [Preferences] → [Miscellaneous] fülnél bekapcsoljuk a külső elérést	50

17. ábra: Hálózati Kontroller beépítése a hálózatba	50

18. ábra: IP cím megadása a Hálózati Kontrollernek	51

19. ábra: Alapértelmezett átjáró beállítása a Hálózati Kontrolleren	51

20. ábra: Port megnyitása LAN-on	51

21. ábra: Elérés tesztelése külső böngészővel	51

22. ábra: Felhasználó létrehozása a Hálózai Kontrollerhez	52

23. ábra: Belépés a létrehozott felhasználóba	52

24. ábra: Hálózati Kontroller Webes felületének főoldala	53

25. ábra: SwR "enable" belépésének adatai	53

26. ábra: SwR hálózati eszközkénti hozzáadása	54

27. ábra: SwR hozzáadva hálózati eszközként és elérhető	54

28. ábra: SwR kapcsolón keresztül elérhető végponti eszközök és adataik	54

29. ábra: Hálózatkonfigurációs Python program	55

30. ábra: Python program kimenete	55

31. ábra: NTP konfigurációja a Linux szerveren	58

32. ábra: Syslog konfigurációja a Linux szerveren	58

33. ábra: NTP ellenőrzése RV forgalomirányítón	58

34. ábra: Ubuntu felhasználó létrehozása	91

35. ábra: Új hálózati kártya hozzáadása	91

36. ábra: Az új hálózati kártya beállítása	92

37. ábra: Az új kártya működésének tesztelése	92

38. ábra: Szükséges csomagok letöltése és telepítése DHCP-hez	93

39. ábra: DHCP konfigurációs fájl (1. rész)	94

40. ábra: DHCP konfigurációs fájl (2. rész)	94

41. ábra: Címosztás interfészhez rendelése	95

42. ábra: Szolgáltatás futásának ellenőrzése	95

43. ábra: Jelenlegi konfiguráció leállítása és törlése	96

44. ábra:Szükséges csomagok letöltése és telepítése DNS-hez(1. rész)	96

45. ábra: Szükséges csomagok letöltése és telepítése DNS-hez(2. rész)	96

46. ábra: 53-as port beállítása	97

47. ábra: DNS szűrő beállítása	97

48. ábra: IP cím beállítása	97

49. ábra: Weboldalunk domain-jének beállítása	97

50. ábra: Szolgáltatás újraindítása és ellenőrzése	98

51. ábra: Szerver beállítása névszerverként (1. rész)	98

52. ábra: Szerver beállítása névszerverként (2. rész)	98

53. ábra: Szükséges csomagok letöltése és telepítése HTTPS-hez	99

54. ábra: Minimális információátadás beállítása	99

55. ábra: Szervernév beállítása	100

56. ábra: Szolgáltatás újraindítása	100

57. ábra: Jogosultág beállítás a mappára	100

58. ábra: Előre elkészített fájlok átimportálása	100

59. ábra: Weboldal konfigurációjának beállítása	101

60. ábra: Új oldal aktiválása és régi oldal deaktiválása, majd a szolgáltatás újraindítása	101

61. ábra: Weboldal elérés tesztelése	101

62. ábra: Trunk portok státusza	102

63. ábra: Ping Fd3 PC és Fvezetoseg PC közt	103

64. ábra: Adott VLAN-okhoz rendelt portok	103

65. ábra: Pingelés EtherChannel-en keresztül	103

66. ábra: HSRP tesztelése	104

67. ábra: Linux IPv4 címkiosztása	104

68. ábra: RF forgalomirányító által kiosztott IP címek	105

69. ábra: IPv6 címkiosztása	105

70. ábra: ASA NAT	105

71. ábra: RR forgalomirányító NAT	106

72. ábra: RV forgalomirányító NAT	106

73. ábra: Dolgozói telefon felcsatlakozik az AP-re	106

74. ábra: Tesztelés előtt és közbeni kép a topológiáról	107

75. ábra: Feszítőfa protokoll összesítő show parancs kimenete	107

76. ábra: SSH belépés Fd3-ról RF-re	108

77. ábra: HTTP weboldal elérés	108

78. ábra: HTTPS weboldal elérés	109

79. ábra: ACL-ek a vezetőség elsődleges forgalomirányítón	109

80. ábra: CBAC működése	110

81. ábra: Belépési adatok és sikeres belépés	110

82. ábra: Syslog naplózás szerver szemszögből	111

83. ábra: Időszinkronizálás NTP-vel	111

84. ábra: VPN kapcsolat tesztelése	112

85. ábra: ISAKMP teszt	112

86. ábra: Hálózatkonfigurációs program kimenete	112

87. ábra: Linux szerver IPv4 címet osztott ki	113

88. ábra: DNS tesztelése	114

89. ábra: Weboldal elérése domain által	114

90. ábra: Weboldal elérhető a szerver IP címe alapján	115

Zoli: 120 oldal

Zalán: 100-110 oldal

Milán: 70 oldal